Problem 11.29

Find H(jw) if its amplitude characteristic is shown in fig 29.

# Suggested Solution

The initial slope of –20dB/dec will cut the odB line at w=40r/s. Therefore the gain is 40. The zeros are at w=50r/s and w=1000r/s. The pole are ar w=0 and there is a double pole at w=400.: