CHAPTER 26 – DC Circuits

1.	(a)	For the current in the single loop, we have

Ia = V/(Ra + r) = (8.50 V)/(68.0  + 0.900 ) = 0.123 A.

For the terminal voltage of the battery, we have

Va = å – Iar = 8.50 V – (0.123 A)(0.900 ) =       8.39 V.

(b)	For the current in the single loop, we have

Ib = V/(Rb + r) = (8.50 V)/(680  + 0.900 ) = 0.0125 A.

For the terminal voltage of the battery, we have

Vb = å – Ibr = 8.50 V – (0.0125 A)(0.900 ) =       8.49 V.

2.	The voltage across the bulb is the terminal voltage of the four cells:

V = IRbulb = 4(å – Ir);

(0.62 A)(12 ) = 4[2.0 V – (0.62 A)r], which gives r =      0.23 .

3.	If we can ignore the resistance of the ammeter, for the single loop we have

I = å/r;

25 A = (1.5 V)/r, which gives r =      0.060 .

4.	We find the internal resistance from

V  = å – Ir;

9.8 V = [12.0 V – (60 A)r], which gives r =      0.037 .

Because the terminal voltage is the voltage across the starter, we have

V = IR;

9.8 V = (60 A)R, which gives R =      0.16 .

5.	When the bulbs are connected in series, the equivalent resistance is

Rseries = ?Ri = 4Rbulb = 4(90 ) =       360 .

When the bulbs are connected in parallel, we find the equivalent resistance from

1/Rparallel = ?(1/Ri) = 4/Rbulb = 4/(90 ), which gives Rparallel =       23 .

6.	(a)	When the bulbs are connected in series, the equivalent resistance is

Rseries = ?Ri = 3R1 + 3R2 = 3(40 ) + 3(80 ) =       360 .

(b)	When the bulbs are connected in parallel, we find the equivalent resistance from

1/Rparallel = ?(1/Ri) = (3/R1) + (3/R2) = [3/(40 )] + [3/(80 )], which gives Rparallel =       8.9 .

7.	If we use them as single resistors, we have

R1 =      25 ;        R2 =      70 .

When the resistors are connected in series, the equivalent resistance is

Rseries = ?Ri = R1 + R2 = 25  + 70  =       95 .

When the resistors are connected in parallel, we find the equivalent resistance from

1/Rparallel = ?(1/Ri) = (1/R1) + (1/R2) = [1/(25 )] + [1/(70 )], which gives Rparallel =       18 .

8.	Because resistance increases when resistors are connected in series, the maximum resistance is

Rseries = R1 + R2 + R3 = 500  + 900  + 1400  = 2800  =       2.80 k.

Because resistance decreases when resistors are connected in parallel, we find the minimum

resistance from

1/Rparallel = (1/R1) + (1/R2) + (1/R3) = [1/(500 )] + [1/(900 )] + [1/(1400 )],

which gives Rparallel =       261 .

9.	The voltage is the same across resistors in parallel, but is less

across a resistor in a series connection.  We connect three 1.0-

resistors in series as shown in the diagram.  Each resistor has

the same current and thus the same voltage:

Vi = @V = @(6.0 V) = 2.0 V.

Thus we can get a 4.0-V output between a and c.

10.	When the resistors are connected in series, as shown in A,

we have

RA = ?Ri = 3R = 3(1.20 k) =       3.60 k.

When the resistors are connected in parallel, as shown in B,

we have

1/RB = ?(1/Ri) = 3/R = 3/(1.20 k), so RB =       0.40 k.

In circuit C, we find the equivalent resistance of the

two resistors in parallel:

1/R1 = ?(1/Ri) = 2/R = 2/(1.20 k), so R1 = 0.60 k.

This resistance is in series with the third resistor,

so we have

RC = R1 + R = 0.60 k + 1.20 k =       1.80 k.

In circuit D, we find the equivalent resistance of the

two resistors in series:

R2 = R + R = 1.20 k + 1.20 k = 2.40 k.

This resistance is in parallel with the third resistor,

so we have

1/RD = (1/R2) + (1/R) = (1/2.40 k) + (1/1.20 k), so RD =       0.80 k.

11.	We can reduce the circuit to a single loop by successively

combining parallel and series combinations.

We combine R1 and R2 , which are in series:

R7	= R1 + R2 = 2.8 k + 2.8 k = 5.6 k.

We combine R3 and R7 , which are in parallel:

1/R8 = (1/R3) + (1/R7) = [1/(2.8 k)] + [1/(5.6 k)],

which gives R8 = 1.87 k.

We  combine R4 and R8 , which are in series:

R9	= R4 + R8 = 2.8 k + 1.87 k = 4.67 k.

We combine R5 and R9 , which are in parallel:

1/R10 = (1/R5) + (1/R9) = [1/(2.8 k)] + [1/(4.67 k)],

which gives R10 = 1.75 k.

We  combine R10 and R6 , which are in series:

Req = R10 + R6 = 1.75 k + 2.8 k =       4.6 k.

12.	(a)	In series the current must be the same for all bulbs.  If all bulbs have the same resistance, they

will have the same voltage:

Vbulb = V/N = (110 V)/8 =       13.8 V.

(b)	We find the resistance of each bulb from

Rbulb = Vbulb/I = (13.8 V)/(0.60 A) =      23 .

The power dissipated in each bulb is

Pbulb = IVbulb = (0.60 A)(13.8 V) =      8.3 W.

13.	For the parallel combination, the total current from the source is

I = NIbulb = 8(0.340 A) = 2.72 A.

The voltage across the leads is

Vleads = IRleads = (2.72 A)(1.5 ) = 4.1 V.

The voltage across each of the bulbs is

Vbulb = V – Vleads = 110 V – 4.1 V = 106 V.

We find the resistance of a bulb from

Rbulb = Vbulb/Ibulb = (106 V)/(0.340 A) =      310 .

The power dissipated in the leads is IVleads and the total power used

is IV, so the fraction wasted is

IVleads/IV = Vleads/V = (4.1 V)/(110 V) = 0.037 =      3.7%.

14.	In series the current must be the same for all bulbs.  If all bulbs have the same resistance, they will have the same voltage:

Vbulb = V/N = (110 V)/8 = 13.8 V.

We find the resistance of each bulb from

Pbulb = Vbulb2/Rbulb ;

7.0 W = (13.8 V)2/Rbulb , which gives Rbulb =      27 .

15.	Fortunately the required resistance is less.  We can reduce the resistance by adding a parallel resistor, which does not require breaking the circuit.  We find the necessary resistance from

1/R = (1/R1) + (1/R2);

1/(320 ) = [1/(480 )] + (1/R2), which gives R2 =       960  in parallel.

16.	The equivalent resistance of the two resistors connected in series is

Rs = R1 + R2 .

We find the equivalent resistance of the two resistors connected in parallel from

1/Rp = (1/R1) + (1/R2),   or   Rp = R1R2/(R1 + R2).

The power dissipated in a resistor is P = V2/R, so the ratio of the two powers is

Pp/Ps = Rs/Rp = (R1 + R2)2/R1R2 = 4.

When we expand the square, we get

R12 + 2R1R2 + R22 = 4R1R2 ,   or  R12 – 2R1R2 + R22 = (R1 – R2)2 = 0, which gives R2 = R1 =      1.6 k.

17.	With the two bulbs connected in parallel, there will be 110 V across each bulb, so the total power will be 75 W + 40 W = 115 W.  We find the net resistance of the bulbs from

P = V 2/R;

115 W = (110 V)2/R, which gives R =       105 .

18.	We can reduce the circuit to a single loop by successively

combining parallel and series combinations.

We combine R1 and R2 , which are in series:

R7 = R1 + R2 = 2.20 k + 2.20 k = 4.40 k.

We combine R3 and R7 , which are in parallel:

1/R8 = (1/R3) + (1/R7) = [1/(2.20 k)] + [1/(4.40 k)],

which gives R8 = 1.47 k.

We  combine R4 and R8 , which are in series:

R9 = R4 + R8 = 2.20 k + 1.47 k = 3.67 k.

We combine R5 and R9 , which are in parallel:

1/R10 = (1/R5) + (1/R9) = [1/(2.20 k)] + [1/(3.67 k)],

which gives R10 = 1.38 k.

We  combine R10 and R6 , which are in series:

Req = R10 + R6 = 1.38 k + 2.20 k = 3.58 k.

The current in the single loop is the current through R6 :

I6 = I = å/Req = (12 V)/(3.58 k) =      3.36 mA.

For VAC we have

VAC = IR10 = (3.36 mA)(1.38 k) = 4.63 V.

This allows us to find I5 and I4 ;

I5 = VAC/R5 = (4.63 V)/(2.20 k) =       2.11 mA;

I4 = VAC/R9 = (4.63 V)/(3.67 k) =       1.26 mA.

For VAB we have

VAB = I4R8 = (1.26 mA)(1.47 k) = 1.85 V.

This allows us to find I3 , I2 , and I1 ;

I3 = VAB/R3 = (1.85 V)/(2.20 k) =       0.84 mA;

I1 = I2 = VAB/R7 = (1.85 V)/(4.40 k) =       0.42 mA.

From above, we have      VAB = 1.85 V.

19.	(a)	When the switch is closed the addition of R2 to the parallel set will decrease the equivalent

resistance, so the current from the battery will increase.  This causes an increase in the voltage

across R1 , and a corresponding decrease across R3 and R4.  The voltage across R2 increases from zero.

Thus we have      V1 and V2 increase; V3 and V4 decrease.

(b)	The current through R1 has increased.  This current is now split into three, so currents through

R3 and R4 decrease.  Thus we have

I1 (= I) and I2 increase; I3 and I4 decrease.

(c)	The current through the battery has increased, so the power output of the battery       increases.

(d)	Before the switch is closed,      I2 = 0.      We find the

resistance for R3 and R4 in parallel from

1/RA = ?(1/Ri) = 2/R3 = 2/(100 ),

which gives RA = 50 .

For the single loop, we have

I 	= I1 = V/(R1 + RA)

= (45.0 V)/(100  + 50 ) =      0.300 A.

This current will split evenly through R3 and R4 :

I3 = I4 =  !I = !(0.300 A) =      0.150 A.

After the switch is closed, we find the

resistance for R2 , R3 , and R4 in parallel from

1/RB = ?(1/Ri) = 3/R3 = 3/(100 ),

which gives RB = 33.3 .

For the single loop, we have

I 	= I1 = V/(R1 + RB)

= (45.0 V)/(100  + 33.3 ) =      0.338 A.

This current will split evenly through R2 , R3 , and R4 :

I2 = I3 = I4 =  @I = @(0.338 A) =      0.113 A.

20.	(a)	When the switch is opened, the removal of a resistor from the parallel set will increase the

equivalent resistance, so the current from the battery will decrease.  This causes a decrease in the

voltage across R1 , and a corresponding increase across R2 .  The voltage across R3 decreases to zero.

Thus we have      V1 and V3 decrease; V2 increases.

(b)	The current through R1 has decreased.  The current through R2 has increased.  The current through

R3 has decreased to zero.  Thus we have

I1 (= I) and I3 decrease; I2 increases.

(c)	Because the current through the battery decreases, the Ir term decreases, so the terminal voltage of

the battery will      increase.

(d)	When the switch is closed, we find the

resistance for R2 and R3 in parallel from

1/RA = ?(1/Ri) = 2/R = 2/(5.50 ),

which gives RA = 2.75 .

For the single loop, we have

I 	= V/(R1 + RA + r)

= (12.0 V)/(5.50  + 2.75  + 0.50 ) = 1.37 A.

For the terminal voltage of the battery, we have

Vab = å – Ir = 12.0 V – (1.37 A)(0.50 ) =       11.3 V.

(e)	When the switch is open, for the single loop,

we have

I	= V/(R1 + R2 + r)

= (12.0 V)/(5.50  + 5.50  + 0.50 ) = 1.04 A.

For the terminal voltage of the battery, we have

Vab = å – Ir = 12.0 V – (1.04 A)(0.50 ) =       11.5 V.

21.	We find the resistance of a bulb from the nominal rating:

Rbulb = Vnominal2/Pnominal = (12.0 V)2/(3.0 W) = 48 .

We find the current through each bulb when connected to the battery from:

Ibulb = V/Rbulb = (11.8 V)/(48 ) = 0.246 A.

Because the bulbs are in parallel, the current through the battery is

I = 2Ibulb = 2(0.246 A) = 0.492 A.

We find the internal resistance from

V  = å – Ir;

11.8 V = [12.0 V – (0.492 A)r], which gives r =      0.4 .

22.	We find the resistance for R1 and R2 in parallel from

1/Rp = (1/R1) + (1/R2) = [1/(3.8 k)] + [1/(2.1 k)],

which gives Rp = 1.35 k.

Because the same current passes through Rp and R3 , the

higher resistor will have the higher power dissipation,

so the limiting resistor is R3 , which will have a power

dissipation of ! W.  We find the current from

P3max = I3max2R3 ;

0.50 W = I3max2(1.8103 ), which gives I3max = 0.0167 A.

The maximum voltage for the network is

Vmax 	= I3max(Rp + R3)

= (0.0167 A)(1.35103  + 1.8103 ) =      53 V.

23.	For the current in the single loop, we have

I	= V/(R1 + R2 + r)

= (9.0 V)/(8.0  + 12.0  + 2.0 ) =       0.41 A.

For the terminal voltage of the battery, we have

Vab = å – Ir = 9.0 V – (0.41 A)(2.0 ) = 8.18 V.

The current in a resistor goes from high to low potential.

For the voltage changes across the resistors, we have

Vbc = – IR2 = – (0.41 A)(12.0 ) = – 4.91 V;

Vca = – IR1 = – (0.41 A)(8.0 ) = – 3.27 V.

For the sum of the voltage changes, we have

Vab + Vbc + Vca = 8.18 V – 4.91 V – 3.27 V = 0.

24.	For the loop, we start at point a:

– IR – å2 – Ir2 + å1 – Ir1 = 0;

– I(6.6 ) – 12 V – I(2 ) + 18 V – I(1 ) = 0,

which gives I = 0.625 A.

The top battery is discharging, so we have

V1 = å1 – Ir1 = 18 V – (0.625 A)(1 ) =       17.4 V.

The bottom battery is charging, so we have

V2 = å2 + Ir2 = 12 V + (0.625 A)(2 ) =       13.3 V.

25.	For the conservation of current at point c, we have

Iin = Iout ;

I1 = I2 + I3 .

For the two loops indicated on the diagram, we have

loop 1:	V1 – I2R2 – I1R1 = 0;

+ 9.0 V – I2(15 ) – I1(22 ) = 0;

loop 2:	V3 + I2R2 = 0;

+ 6.0 V + I2(15 ) = 0.

When we solve these equations, we get

I1 = 0.68 A, I2 = – 0.40 A,       I3 = 1.08 A.

Note that I2 is opposite to the direction shown.

26.	For the conservation of current at point c, we have

Iin = Iout ;

I1 = I2 + I3 .

When we add the internal resistance terms for the two

loops indicated on the diagram, we have

loop 1:	V1 – I2R2 – I1R1 – I1r1 = 0;

+ 9.0 V – I2(15 ) – I1(22 ) – I1(1.2 ) = 0;

loop 2:	V3 + I2R2 – I3r3 = 0;

+ 6.0 V + I2(15 ) – I3(1.2 ) = 0.

When we solve these equations, we get

I1 = 0.60 A, I2 = – 0.33 A,       I3 = 0.93 A.

27.	For the conservation of current at point b, we have

Iin = Iout ;

I1 + I3 = I2 .

For the two loops indicated on the diagram, we have

loop 1:	å1 – I1R1 – I2R2 = 0;

+ 9.0 V – I1(15 ) – I2(20 ) = 0;

loop 2:	– å2 + I2R2 + I3R3 = 0;

– 12.0 V + I2(20 ) + I3(40 ) = 0.

When we solve these equations, we get

I1 = 0.18 A right, I2 = 0.32 A left, I3 = 0.14 A up.

28.	For the conservation of current at point b, we have

Iin = Iout ;

I1 + I3 = I2 .

When we add the internal resistance terms for the two

loops indicated on the diagram, we have

loop 1:	å1 – I1r1 – I1R1 – I2R2 = 0;

+ 9.0 V – I1(1.0 ) – I1(15 ) – I2(20 ) = 0;

loop 2:	– å2 + I3r2 + I2R2 + I3R3 = 0;

– 12.0 V + I3(1.0 ) + I2(20 ) + I3(40 ) = 0.

When we solve these equations, we get

I1 = 0.17 A right, I2 = 0.31 A left, I3 = 0.14 A up.

29.	When we include the current through the battery, we have

six unknowns.  For the conservation of current, we have

junction a: 	I = I1 + I2 ;

junction b: 	I1 = I3 + I5 ;

junction d: 	I2 + I5 = I4 .

For the three loops indicated on the diagram, we have

loop 1:	– I1R1 – I5R5 + I2R2 = 0;

– I1(20 ) – I5(10 ) + I2(25 ) = 0;

loop 2:	– I3R3 + I4R4 + I5R5 = 0;

– I3(2 ) + I4(2 ) + I5(10 ) = 0;

loop 3:	+ å – I2R2 – I4R4 = 0.

+ 6.0 V – I2(25 ) – I4(2 ) = 0.

When we solve these six equations, we get

I1 = 0.274 A, I2 = 0.222 A, I3 = 0.266 A, I4 = 0.229 A, I5 = 0.007 A, I = 0.496 A.

We have carried an extra decimal place to show the agreement with the junction equations.

30.	When the 20- resistor is shorted, points a and b become

the same point and we lose I1 .

For the conservation of current, we have		junction a: 	I = I2 + I3 + I5 ;		junction d: 	I2 + I5 = I4 ;	For the three loops indicated on the diagram, we have		loop 1:	+ I2R2 – I5R5 = 0;				+ I2(25 ) – I5(10 ) = 0;		loop 2:	– I3R3 + I4R4 + I5R5 = 0;				– I3(2 ) + I4(2 ) + I5(10 ) = 0;		loop 3 (around outside):	+ å – I3R3 = 0.				+ 6.0 V – I3(2 ) = 0;	When we solve these five equations, we get		I2 = 0.19 A, I3 = 3.00 A, I4 = 0.66 A, I5 = 0.47 A, I = 3.66 A.	Thus the current through the 10- resistor is      0.47 A down.31.	If we assume the current in R4 is to the right, we have		Vcd = I4R4 = (3.50 mA)(4.0 k) = 14.0 V.	We can now find the current in R8 :		I8 = Vcd/R8 = (14.0 V)/(8.0 k) = 1.75 mA.	From conservation of current at the junction c, we have

I = I4 + I8 = 3.50 mA + 1.75 mA = 5.25 mA.

If we go clockwise around the outer loop, starting at a,

we have

Vba – IR5 – I4R4 – å – Ir = 0,   or

Vba = (5.25 mA)(5.0 k) + (3.50 mA)(4.0 k) + 12.0 V + (5.25 mA)(1.010–3 k) =     52 V.

If we assume the current in R4 is to the left, all currents are reversed, so we have

Vdc = 14.0 V;   I8 = 1.75 mA,   and   I = 5.25 mA.

If we go counterclockwise around the outer loop, starting at a, we have

– Ir + å – I4R4 – IR5 + Vab = 0,   or   Vba = – Vab = – Ir + å – I4R4 – IR5 ;

Vba = – (5.25 mA)(1.010–3 k) + 12.0 V – (3.50 mA)(4.0 k) – (5.25 mA)(5.0 k) =     – 28 V.

The negative value means the battery is facing the other direction.

32.	The given current is I2 = + 0.90 A.  For the conservation of current at point a, we have

I2 + I3 = I1 ,   or   0.90 A + I3 = I1 .

We assume r1 = r2 = r3 = r4 = 1.0 .  For a loop CCW around the outside, we have

å1 – I1r1 – I1R3 – I3R4 + å3 – I3r3 – I3R5 – I1R1 = 0;

12.0 V – I1(1.0 ) – I1(8.0 ) – (I1 – 0.90 A)(15 ) + 6.0 V –

(I1 – 0.90 A)(1.0 ) – (I1 – 0.90 A)(18 ) – I1(12 ) = 0,

which gives I1 = 0.884 A.

For the top loop indicated on the diagram, we have

loop 1:	å1 – I1r1 – I1R3 + å2 – I2r2 – I2R – I1R1 = 0;

+ 12.0 V – (0.884 A)(1.0 ) – (0.884 A)(8.0 ) + 12.0 V –

(0.90 A)(1.0 ) – (0.90 A)R – (0.884 A)(12 ) = 0,

which gives R =       5.0 .

33.	The given current is I2 = – 0.30 A.  For the conservation of current at point a, we have

I2 + I3 = I1 ,   or   – 0.30 A + I3 = I1 .

For the top loop indicated on the diagram, we have

loop 1:	å1 – I1r1 – I1R3 + å2 – I2r2 – I2R2 – I1R1 = 0;

+ 12.0 V – I1(1.0 ) – I1(8.0 ) + 12.0 V –

(– 0.30 A)(1.0 ) – (– 0.30 A)(10 ) – I1(12 ) = 0,

which gives I1 = 1.30 A.

Thus we have

I3 = I1 – I2 = 1.30 A – (– 0.30 A) = 1.60 A.

For the bottom loop indicated on the diagram, we have

loop 2:	å – I3r – I3R5 + I2R2 – å2 + I2r2 – I3R4 = 0;

å – (1.60 A)(1.0 ) – (1.60 A)(18 ) + (– 0.30 A)(10 ) –

12.0 V + (– 0.30 A)(1.0 ) – (1.60 A)(15 ) = 0,

which gives å =      70 V.

34.	For the conservation of current at point a, we have

I2 + I3 = I1 .

For the two loops indicated on the diagram, we have

loop 1:	å1 – I1r1 – I1R3 + å2 – I2r2 – I2R2 – I1R1 = 0;

+ 12.0 V – I1(1.0 ) – I1(8.0 ) + 12.0 V –

I2(1.0 ) – I2(10 ) – I1(12 ) = 0;

loop 2:	å3 – I3r3 – I3R5 + I2R2 – å2 + I2r2 – I3R4 = 0;

+ 6.0 V – I3(1.0 ) – I3(18 ) + I2(10 ) –

12.0 V + I2(1.0 ) – I3(15 ) = 0.

When we solve these equations, we get

I1 = 0.77 A, I2 = 0.71 A, I3 = 0.055 A.

For the terminal voltage of the 6.0-V battery, we have

Vfe = å3 – I3r3 = 6.0 V – (0.055 A)(1.0 ) =        5.95 V.

35.	The lower loop equation becomes

loop 2:	å3 – I3r3 + I2R2 – å2 + I2r2 – I3R4 = 0;

+ 6.0 V – I3(1.0 ) + I2(10 ) – 12.0 V + I2(1.0 ) – I3(15 ) = 0.

The other equations are the same:

I2 + I3 = I1 .

loop 1:	å1 – I1r1 – I1R3 + å2 – I2r2 – I2R2 – I1R1 = 0;

+ 12.0 V – I1(1.0 ) – I1(8.0 ) + 12.0 V – I2(1.0 ) – I2(10 ) – I1(12 ) = 0.

When we solve these equations, we get      I1 = 0.783 A,       I2 = 0.686 A, I3 = 0.097 A.

We have carried an extra decimal place to show the agreement with the junction equations.

36.	(a)	On the diagram, we show the potential difference applied

between points a and c.  Because all of the resistors are the

same, symmetry means that the three currents leaving

point a must be the same three currents entering point c.

This means that there is no current in the resistor between

points b and d, which can be removed without changing the

currents.  When we redraw the circuit, we see that we have

three parallel branches between points a and c.

We find the equivalent resistance from

1/Rac = (1/R) + (1/2R) + (1/2R),

which gives Rac =      R/2.

(b)	If we apply a potential difference between points a and b,

the same symmetry exists, so all currents leaving a must enter b.

Consequently there is no current in R, which can be removed.

When we redraw the circuit, we get one identical to that in

part (a), so the equivalent resistance is      R/2.

37.	We find the equivalent resistance for the parallel connection from

1/Rparallel = (1/R) + (1/R) + … = n/R,   or   Rparallel = R/n.

We find the equivalent resistance for the series connection from

Rseries = R + R + … = nR.

The  power transformation is P = IV = V 2/R.  If we form the ratio at constant voltage for the two connections, we get

Pseries/Pparallel = Rparallel/Rseries = (R/n)/nR = 1/n2.

38.	(a)	For the conservation of current at point d, we have

I4 = I1 + I3 .

For the conservation of current at point e, we have

I2 = I3 + I5 .

For the three loops indicated on the diagram, we have

loop 1:	å1 – I1R1 – I4R4 = 0;

+ 14 V – (I4 – I3)(10 k) – I4(12 k) = 0;

loop 2:	å2 – I2R2 – I5R3 = 0;

+ 18 V – (I3 + I5)(15 k) – I5(20 k) = 0;

loop 3:	å3 + I5R3 – I4R4 = 0;

+ 12 V + I5(20 k) – I4(12 k) = 0.

We now have three equations for three currents.  When we solve these and use the junction

equations, we get

I1 = 0.0665 mA,       I2 = 1.111 mA, I3 = 1.045 mA, I4 = 1.111 mA, I5 = 0.0663 mA.

(b)	If we start at point b and add the potential drops, we have

Vb – I2R2 + I1R1 = Va ,   or  Vab = – (1.111 mA)(15 k) + (0.0665 mA)(10 k) =      – 16 V.

39.	(a)	On the diagram, we show the potential difference applied

between points a and c, and the six currents.

For the conservation of current we have

at point a:    I = I1 + I2 + I3 ;				(1)

at point b:    I3 = I4 + I5 ;

at point d:    I2 + I4 = I6 .

For a loop CW around the right triangle, we have

– I4R – I6R  + I5R = 0;

– I4R – (I2 + I4)R  + (I3 – I4)R = 0,   or

(2R + R )I4 = I3R – I2R . 					(2)

For a loop CW around the left triangle, we have

– I2R + I4R + I3R = 0,   or   I4 = I2 – I3 .			(3)

For a loop CW around the bottom triangle, we have

– I3R – I5R + I1R = 0;

– I3R – (I3 – I4)R + I1R = 0,   or   I4 = 2I3 – I1 .		(4)

When we combine (3) and (4), we get

3I3 = I1 + I2 .								(5)

When we combine (2) and (3), we get

(2R + R )I2 – (2R + R )I3 = I3R – I2R ,   or

I3 = 2(R + R )I2/(3R + R ).				(6)

When we combine (5) and (6), we get

I2 = (3R + R )I1/(3R + 5R ).				(7)

When we combine (6) and (7), we get

I3 = 2(R + R )I1/(3R + 5R ).				(8)

When we use (7) and (8) in (1), we get

I = 8(R + R )I1/(3R + 5R ).

Because I1 = Vac/R, for the equivalent resistance we have

Req = Vac/I = I1R/I =       R(3R + 5R )/8(R + R ).

(b)	If we apply a potential difference between points a and b,

all currents leaving a must enter b.  From the symmetry

I1 = I2 and I4 = I5 , so I2 = – I4 and I1 = – I5 .  Consequently there

is no current in R, which can be removed.  When we redraw

the circuit, we see that we have three parallel branches

between points a and b.  We find the equivalent resistance from

1/Rab = (1/R) + (1/2R) + (1/2R),

which gives Rab =      R/2.

40.	(a)	On the diagram, we have used the symmetry to reduce the number of currents to five, as labeled.

For the conservation of current we have

at point a:    I = 2I1 + I2 ; 				(1)

at point c:    I1 = I3 + I4 ;				(2)

at point d:    I5 = 2I4 .

For a loop CW around the bottom of the cube, we have

– I4R – I5R– I4R  + I3R = 0;

– 2I4R – (2I4)R  + I3R = 0,   or   I3 = 4I4 .		(3)

For a loop CW around the top of the cube, we have

– I1R – I3R– I1R  + I2R = 0,   or   I2 = 2I1 + I3 .		(4)

When we combine (2) and (3), we get

I3 = 4I1/5.								(5)

When we combine (4) and (5), we get

I2 = 14I1/5.								(6)

When we combine (1) and (6), we get

I = 24I1/5.								(7)

Because I2 = Vab/R, for the equivalent resistance we have

Req = Vab/I = I2R/I = (14I1/5)R/(24I1/5) =       7R/12.

(b)	On the diagram, we have used the symmetry to reduce the number of currents to three, as labeled.

Note that there are two resistors with zero current.

For the conservation of current we have

at point a:    I = 2I1 + I2 ; 				(1)

at point d:    I2 = 2I3 .				(2)

For a loop CW around the top of the cube, we have

– I2R – I3R+ 0 + I1R = 0,   or   I1 = I2 + I3 .		(3)

When we combine (2) and (3), we get

I2 = 2I1/3.								(4)

When we combine (1) and (4), we get

I = 8I1/3.								(5)

Because I1 = Vac/2R, for the equivalent resistance we have

Req = Vac/I = I1(2R)/I = 2I1R/(8I1/3) =       3R/4.

(c)	On the diagram, we have used the symmetry to reduce the number of currents to two, as labeled.

For the conservation of current we have

at point a:    I = 3I1 ;

at point c:    2I2 = I1 .

For a path from a to d we have

Vad = I1R + I2R + I1R = (2I1 + I1/2)R = 5I1R/2.

For the equivalent resistance we have

Req = Vad/I = (5I1R/2)/3I1 =       5R/6.

41.	(a)	We find the capacitance from

 = RC;

5510–6 s = (15103 )C,

which gives C = 3.710–9 F =       3.7 nF.

(b)	The voltage across the capacitance will increase to the final

steady state value.  The voltage across the resistor will start

at the battery voltage and decrease exponentially:

VR = å e – t/ ;

16.0 V = (24.0 V)e – t/(55 s),   or   t/(55 s) = ln(24.0 V/16.0 V) = 0.405, which gives t =      22 s.

42.	The time constant of the circuit is

 = RC = (6.7103 )(6.010–6 F) = 0.0402 s.

The capacitor voltage will decrease exponentially:

VC = V0 e – t/ ;

0.01V0 = V0 e – t/(0.0402 s),   or   t/(0.0402 s) = ln(100) = 4.61,

which gives t =      0.19 s.

43.	The charge on the capacitor increases with time to a final charge Q0 :

Q = Q0 (1 – e – t/ ).

When we express the stored energy in terms of charge we have

U = !CV2 = !Q2/C = !(Q02/C)(1 – e – t/ )2 = Umax(1 – e – t/ )2.

We find the time to reach half the maximum from

! = (1 – e – t/ )2,   or   e – t/  = 1 – 1/v2, which gives       t = 1.23.

44.	We find the equivalent capacitance from

1/Ceq = (1/C1) + (1/C2) = [1/(6.0 F)] + [1/(6.0 F)],

which gives Ceq = 3.0 F.

We find the equivalent resistance from

Req = R1 + R2 =2.2 k + 2.2 k = 4.4 k.

The time constant of the circuit is

 = ReqCeq = (4.4 k)(3.0 F) = 13.2 ms.

Initially there is no charge on the capacitors, so the applied voltage is across the equivalent resistance:

å = I0Req ;

12.0 V = I0(4.4 k), which gives I0 = 2.73 mA.

The current will decrease exponentially:

I = I0 e – t/ ;

1.50 mA = (2.73 mA)e – t/ , which gives t = 0.60 = (0.60)(13.2 ms) =     7.9 ms.

45.	(a)	For the conservation of current at point a, we have

I1 = I2 + I3 .

For the loop on the right, we have

+ I2R2 – Q/C = 0,  or  I2R2 = Q/C.

For the outside loop, we have

å – I1R1 – Q/C = 0,  or  I1R1 = å – Q/C.

The current I3 is charging the capacitor: I3 = dQ/dt.

When we use these results in the junction equation, we get

(å – Q/C)/R1 = (Q/R2C) + dQ/dt, which becomes

å = R1 dQ/dt + (R1 + R2)Q/R2C.

This has the same form as the simple RC circuit:

å = R dQ/dt + Q/C,

if we replace R with R1 , and C with R2C/(R1 + R2).

Thus the time constant is

 = R1R2C/(R1 + R2).

(b)	After a long time, the current through the capacitor will be zero and it will have its maximum

charge.  The current through the resistors will be

I = å/(R1 + R2).

The voltage across the capacitor will be the voltage across R2:  V = IR2 .  Thus the charge on the

capacitor will be

Qmax = IR2C =       åR2C/(R1 + R2).

46.	In the steady state there is no current through the capacitors.

(a)	The current through the resistors is

I = Vcd/(R1 + R2) = (24 V)/(8.8  + 4.4 ) = 1.82 A.

The potential at point a is

Va = Vad = IR2 = (1.82 A)(4.4 ) =      8.0 V.

(b)	We find the equivalent capacitance of the two series capacitors:

1/C = (1/C1) + (1/C2) = [1/(0.48 F)] + [1/(0.24 F)],

which gives C = 0.16 F.

We find the charge on each of the two in series:

Q1 = Q2 = Q = CVcd = (0.16 F)(24 V) = 3.84 C.

The potential at point b is

Vb = Vbd = Q2/C2 = (3.84 C)/(0.24 F) =       16 V.

(c)	With the switch closed, the current is the same.  Point b must have the same potential as point a:

Vb = Va =       8.0 V.

(d)	We find the charge on each of the two capacitors, which are no longer in series:

Q1 = C1Vcb = (0.48 F)(24 V – 8.0 V) = 7.68 C;

Q2 = C2Vbd = (0.24 F)(8.0 V) = 1.92 C.

When the switch was open, the net charge at point b was zero, because the charge on the negative

plate of C1 had the same magnitude as the charge on the positive plate of C2.  With the switch

closed, these charges are not equal.  The net charge at point b is

Qb = – Q1 + Q2 = – 7.68 C + 1.92 C =       – 5.8 C,      which flowed through the switch.

47.	Because we have no simple series or parallel connections, we analyze the circuit.  On the diagram, we show the potential difference applied between points c and d, and the four currents.

For the conservation of current at points c and d, we have

I = I1 + I3 = I2 + I4 . 						(1)

The current I3 is charging the capacitor C1 : I3 = dQ1/dt.

The current I4 is charging the capacitor C2 : I4 = dQ2/dt.

For a CW loop acba, we have

I1R1 – Q1/C1 = 0,    or

Q1/C1 = (I – I3)R1 = (I – dQ1/dt)R1 . 			(2)

For a CW loop dabd, we have

I2R2 – Q2/C2 = 0, or

Q2/C2 = (I – I4)R2 = (I – dQ2/dt)R2 . 			(3)

For the path cbd, we have

Vcd = (Q1/C1) + (Q2/C2).					(4)

If we differentiate this, we get

dVcd/dt = 0 = (1/C1)(dQ1/dt) + (1/C2)(dQ2/dt),   or

dQ2/dt = – (C2/C1) dQ1/dt.					(5)

When we combine (3) and (5), we get

Q2/C2 = R2[I + (C2/C1) dQ1/dt].				(6)

When we combine (2) and (6), we get

Q2/C2 = (R2/R1C1)Q1 + [R2(C1 + C2)/C1] dQ1/dt.	(7)

When we combine (4) and (7), we get

Vcd =  [(R1 + R2)/R1C1]Q1 + [R2(C1 + C2)/C1] dQ1/dt.

This has the same form as the simple RC circuit:

å = R dQ/dt + Q/C,

with R = R2(C1 + C2)/C1 , and C = R1C1/(R1 + R2).

Thus the time constant is

 	= R1R2(C1 + C2)/(R1 + R2)

= (8.8 )(4.4 )(0.48 F + 0.24 F)/(8.8  + 4.4 ) =       2.1 s.

48.	We find the resistance of the voltmeter from

R = (sensitivity)(scale) = (50,000 /V)(250 V) = 1.25107  =       12.5 M.

49.	We find the current for full-scale deflection of the ammeter from

I = Vmax/R = Vmax/(sensitivity)Vmax = 1/(20,000 /V) = 5.010–5 A =       50 A.

50.	(a)	We make an ammeter by putting a resistor in parallel with

the galvanometer.  For full-scale deflection, we have

Vmeter = IGr = IsRs ;

(5010–6 A)(30 ) = (30 A – 5010–6 A)Rs ,

which gives Rs =       5010–6  in parallel.

(b)	We make a voltmeter by putting a resistor in series with

the galvanometer.  For full-scale deflection, we have

Vmeter = I(Rx + r) = IG(Rx + r);

1000 V = (5010–6 A)(Rx + 30 ),

which gives Rx = 20106  =      20 M in series.

51.	(a)	The current for full-scale deflection of the galvanometer is

I = 1/(sensitivity) = 1/(35 k/V) = 2.8610–2 mA.

We make an ammeter by putting a resistor in parallel with

the galvanometer.  For full-scale deflection, we have

Vmeter = IGr = IsRs ;

(2.8610–5 A)(20.0 ) = (2.0 A – 2.8610–5 A)Rs ,

which gives Rs =       2.910–4  in parallel.

(b)	We make a voltmeter by putting a resistor in series with

the galvanometer.  For full-scale deflection, we have

Vmeter = I(Rx + r) = IG(Rx + r);

1.00 V = (2.8610–5 A)(Rx + 20 ),

which gives Rx = 3.5104  =      35 k in series.

52.	We can treat the milliammeter as a resistor

and find its resistance from

1/RA = (1/Rs) + (1/r) = (1/0.20 ) + (1/30 ),

which gives RA = 0.199 .

We make a voltmeter by putting a resistor in series with

the galvanometer.  For full-scale deflection, we have

Vmeter = I(Rx + RA);

10 V = (2010–3 A)(Rx + 0.199 ),

which gives Rx = 5.0102  =      0.50 k in series.

The sensitivity of the voltmeter is

Sensitivity = (500 )/(10 V) =      50 /V.

53.	Before connecting the voltmeter, the current in the series circuit is

I0 = å/(R1 + R2) = (45 V)/(37 k + 28 k) = 0.692 mA.

The voltages across the resistors are

V01 = I0R1 = (0.692 mA)(37 k) = 25.6 V;

V02 = I0R2 = (0.692 mA)(28 k) = 19.4 V.

When the voltmeter is across R1 , we find the equivalent

resistance of the pair in parallel:

1/RA = (1/R1) + (1/RV) = (1/37 k) + (1/100 k),

which gives RA = 27.0 k.

The current in the circuit is

I1 = å/(RA + R2) = (45 V)/(27 k + 28 k) = 0.818 mA.

The reading on the voltmeter is

VV1 = I1RA = (0.818 mA)(27.0 k) = 22.1 V =      22 V.

When the voltmeter is across R2 , we find the equivalent

resistance of the pair in parallel:

1/RB = (1/R2) + (1/RV) = (1/28 k) + (1/100 k),

which gives RB = 21.9 k.

The current in the circuit is

I2 = å/(R1 + RB) = (45 V)/(37 k + 22 k) = 0.764 mA.

The reading on the voltmeter is

VV2 = I2RB = (0.764 mA)(21.9 k) = 16.7 V =      17 V.

We find the percent inaccuracies introduced by the meter:

(25.6 V – 22.1 V)(100)/(25.6 V) =      14% low;

(19.4 V – 16.7 V)(100)/(19.4 V) =      14% low.

54.	We find the voltage of the battery from the series circuit

with the ammeter in it:

å 	= IA(RA + R1 + R2)

= (4.2510–3 A)(60  + 700  + 400 ) = 4.93 V.

Without the meter in the circuit, we have

å = I0(R1 + R2);

4.93 V = I0(700  + 400 ),

which gives I0 = 4.4810–3 A =      4.48 mA.

55.	We find the equivalent resistance of the voltmeter in parallel

with one of the resistors:

1/R = (1/R1) + (1/RV) = (1/9.0 k) + (1/11.5 k),

which gives R = 5.05 k.

The current in the circuit, which is read by the ammeter, is

I 	= å/(r + RA + R + R2)

= (12.0 V)/(1.0  + 0.50  + 5.05 k + 9.0 k)

=       0.85 mA.

The reading on the voltmeter is

Vab = IR = (0.85 mA)(5.05  k) =       4.3 V.

56.	When the voltmeter is across R1 , for the junction b, we have

I1A + I1V = I1 ;

[(5.5 V)/R1] + [(5.5 V)/(15.0 k)] = (12.0 V – 5.5 V)/R2 ;

[(5.5 V)/R1] + 0.367 mA = (6.5 V)/R2 .

When the voltmeter is across R2 , for the junction e, we have

I2 = I2A + I2V ;

(12.0 V – 4.0 V)/R1  = [(4.0 V)/R2] + [(4.0 V)/(15.0 k)];

[(8.0 V)/R1] = [(4.0 V)/R2] + 0.267 mA.

We have two equations for two unknowns, with the results:

R1 = 9.4 k,      and      R2 = 6.8 k.

57.	The resistance of the voltmeter is

RV = (sensitivity)(scale) = (1000 /V)(3.0 V) = 3.0103  = 3.0 k.

We find the equivalent resistance of the resistor and the voltmeter from

1/Req = (1/R) + (1/RV),   or

Req = RRV/(R + RV) = (8.4 k)(3.0 k)/(8.4 k + 3.0 k) = 2.21 k.

The voltmeter measures the voltage across this equivalent resistance,

so the current in the circuit is

I = Vab/Req = (2.0 V)/(2.21 k) = 0.905 mA.

For the series circuit, we have

å = I(R + Req) = (0.905 mA)(8.4 k + 2.21 k) =      9.6 V.

58.	We find the resistances of the voltmeter scales:

RV100 	= (sensitivity)(scale)

= (20,000 /V)(100 V) = 2.0106  = 2000 k;

RV30 	= (sensitivity)(scale)

= (20,000 /V)(30 V) = 6.0105  = 600 k.

The current in the circuit is

I = (Vab/RV) + (Vab/R1).

For the series circuit, we have

å = Vab + IR2 .

When the 100-volt scale is used, we have

I = [(25 V)/(2000 k)] + [(25 V)/(120 k)] = 0.221 mA.

å = 25 V + (0.221 mA)R2 .

When the 30-volt scale is used, we have

I = [(23 V)/(600 k)] + [(23 V)/(120 k)] = 0.230 mA.

å = 23 V + (0.230 mA)R2 .

We have two equations for two unknowns, with the results:  å = 74.1 V, and      R2 = 222 k.

Without the voltmeter in the circuit, we find the current:

å = I (R1 + R2);

74.1 V = I (120 k + 222 k), which gives I = 0.217 mA.

Thus the voltage across R1 is

Vab = I R1 = (0.217 mA)(120 k) =      26 V.

59.	(a)	In circuit 1 the voltmeter is placed in parallel with the resistor,

so we find their equivalent resistance from

1/Req1 = (1/R) + (1/RV).

The ammeter measures the current through this equivalent

resistance and the voltmeter measures the voltage across

this equivalent resistance, so we have

Req1 = V/I.

Thus we have

I/V = (1/R) + (1/RV),  or   1/R = (I/V) – (1/RV).

(b)	In circuit 2 the ammeter is placed in series with the resistor,

so we find their equivalent resistance from

Req2 = R + RA .

The ammeter measures the current through this equivalent

resistance and the voltmeter measures the voltage across

this equivalent resistance, so we have

Req2 = V/I = R + RA ,  or   R = (V/I) – RA .

60.	We find the temperature of the junction from

å = k(T – T0);

1.7210–3 V = (4010–6 V/C°)(T – 25°C), which gives T =      68°C.

61.	We assume the lowest emf represents the uncertainty in a reading.  We find the uncertainty in the temperature from

?å = k ?T;

0.50 V = (14 V/C°) ?T, which gives ?T =       3.610–2 C°.

62.	(a)	We let m be the mass density.  Because the mass is constant, when a wire is stretched, its area

must change.  If we represent the small changes as differentials, we have			A = m/mL,   and  dA = – (m/mL2) dL ,   or   dA/A = – dL/L.		The resistance will change from the change in length and the change in area:			R = L/A,    and   dR = (/A) dL – (L/A2) dA,   or			dR/R = dL/L – dA/A = 2 dL/L.		Thus the fractional change in resistance is proportional to the fractional change in length.	(b)	We find the values of the resistances from			Rx0 = (R2/R1)R30 = (1.4800)(40.700 ) = 60.236 ;			Rx = (R2/R1)R3 = (1.4800)(40.736 ) = 60.289 .		We find the change in width from			K = (?R/Rx0)/(?L/L);			1.8 = [(60.289  – 60.236 )/(60.236 )]/(?L/4.5 mm), which gives ?L =       2.210–3 mm.63.	The voltage is the same across resistors in parallel, but is less 	across a resistor in a series connection.  We connect two resistors 	in series as shown in the diagram.  Each resistor has the same 	current:		I = V/(R1 + R2) = (6.0 V)/(R1 + R2).

If the desired voltage is across R1 , we have

Vab = IR1 = (6.0 V)R1/(R1 + R2);

0.25 V = (6.0 V)R1/(R1 + R2) = (6.0 V)/[1 + (R2/R1)],

which gives R2/R1 = 23.

When the body is connected across ab, we want very negligible current through the body, so the potential difference does not change.  This requires Rbody = 2000  » R1 .  If we also do not want a large current from the battery, a possible combination is

R1 = 4 , R2 = 92 .

64.	Because the voltage is constant and the power is additive,

we can use two resistors in parallel.  For the lower ratings,

we use the resistors separately; for the highest rating, we

use them in parallel.  The rotary switch shown allows

the B contact to successively connect to C and D.  The A

contact connects to C and D for the parallel connection.

We find the resistances for the three settings from

P = V 2/R;

50 W = (120 V)2/R1 , which gives R1 = 288 ;

100 W = (120 V)2/R2 , which gives R2 = 144 ;

150 W = (120 V)2/R3 , which gives R3 = 96 .

As expected, for the parallel arrangement we have

1/Req = (1/R1) + (1/R2);

1/Req = [1/(288 )] + [1/(144 )], which gives Req = 96  = R3 .

Thus the two required resistors are      288 , 144 .

65.	The voltage drop across one of the wires is

Vdrop = IR = (3.0 A)(0.0065 /m)(115 m) =       2.2 V.

The applied voltage at the apparatus is

V = V0 – 2Vdrop = 120 V – 2(2.2 V) =      116 V.

66.	We find the current through the patient (and nurse) from the series circuit:

I 	= V/(Rmotor + Rbed + Rnurse + Rpatient)

= (220 V)/(104  + 0 + 104  + 104 ) = 7.310–3 A =       7.3 mA.

67.	The time between firings is

t = (60 s)/(72 beats) = 0.833 s.

In this time the capacitor reaches 45% of maximum, so we have

V = V0(1 – e – t/) = 0.45V0 , which gives e – t/ = 0.55,   or    = t/ln(1.82) = (0.833 s)/ln(1.82) = 1.39 s.

We find the required resistance from

 = RC;

1.39 s = R(7.5 F), which gives R =      0.19 M.

68.	We find the required current for the hearing aid from

P = IV;

2 W = I(4.0 V), which gives I= 0.50 A.

With this current the terminal voltage of the three mercury cells would be

Vmercury = 3(åmercury – Irmercury) = 3[1.35 V – (0.50 A)(0.030 )] = 4.01 V.

With this current the terminal voltage of the three dry cells would be

Vdry = 3(ådry – Irdry) = 3[1.5 V – (0.50 A)(0.35 )] = 3.98 V.

Thus the      mercury cells would have a higher terminal voltage.

69.	(a)	We find the current from

I1 = V/Rbody = (110V)/(1100 ) =      0.10 A.

(b)	Because the alternative path is in parallel, the current is the same:      0.10 A.

(c)	The current restriction means that the voltage will change.  Because the voltage will be the same

across both resistances, we have

I2R2 = IbodyRbody ;

I2(40 ) = Ibody(1100 ),   or   I2 = 27.5Ibody .

For the sum of the currents, we have

I2 + Ibody = 28.5Ibody = 1.5 A, which gives Ibody = 5.310–2 A =      53 mA.

70.	The resistance of the platinum wire is

Rx = (R2/R1)R3 = (46.0 /38.0 )(3.48 ) = 4.21 .

We find the length from

R 	= L/A

= (10.610–8 · m)L/p(0.46010–3 m)2, which gives L =      26.4 m.

71.	For the conservation of current at point b, we have

I = I1 + I2 .

For the two loops indicated on the diagram, we have

loop 1:	å1 – I1r1 – IR = 0;

+ 2.0 V – I1(0.10 ) – I(4.0 ) = 0;

loop 2:	å2 – I2r2 – IR = 0;

+ 3.0 V – I2(0.10 ) – I(4.0 ) = 0.

When we solve these equations, we get

I1 = – 4.69 A, I2 = 5.31 A, I = 0.62 A.

For the voltage across R we have

Vab = IR = (0.62 A)(4.0 ) =       2.5 V.

Note that one battery is charging the other with a significant current.

72.	Because the two sides of the circuit are identical, we find the resistance from the time constant:

 = RC;

3.0 s = R(3.0 F), which gives R =       1.0 M.

73.	The terminal voltage of a discharging battery is

V = å – Ir.

For the two conditions, we have

40.8 V = å – (7.40 A)r;

44.5 V = å – (2.20 A)r.

We have two equations for two unknowns, with the solutions: å =       46.1 V,      and r =       0.71 .

74.	One arrangement is to connect N resistors in series.  Each resistor will have the same power, so we need

N = Ptotal/P = 5 W/! W = 10 resistors.

We find the required value of resistance from

Rtotal = NRseries ;

1.2 k = 10Rseries , which gives Rseries = 0.12 k.

Thus we have     10  0.12-k resistors in series.

Another arrangement is to connect N resistors in parallel.  Each resistor will again have the same power, so we need the same number of resistors:  10.

We find the required value of resistance from

1/Rtotal = ?(1/Ri) = N/Rparallel ;

1/1.2 k = 10/Rparallel , which gives Rparallel = 12 k.

Thus we have     10  12-k resistors in parallel.

75.	The resistance along the potentiometer is proportional to the

length, so we find the equivalent resistance between points b and c:

1/Req = (1/xRpot) + (1/Rbulb),   or

Req = xRpotRbulb/(xRpot + Rbulb).

We find the current in the loop from

I = V/[(1 – x)Rpot + Req].

The potential difference across the bulb is

Vbc = IReq , so the power expended in the bulb is

P = Vbc2/Rbulb .

(a)	For x = 1.00 we have

Req = (1.00)(100 )(200 )/[(1.00)(100 ) + 200 ] = 66.7 .

I = (120 V)/[(1 – 1.00)(100 ) + 66.7 ] = 1.80 A.

Vbc = (1.80 A)(66.7 ) = 120 V.

P = (120 V)2/(200 ) =      72.0 W.

(b)	For x = 0.50 we have

Req = (0.50)(100 )(200 )/[(0.50)(100 ) + 200 ] = 40.0 .

I = (120 V)/[(1 – 0.50)(100 ) + 40.0 ] = 1.33 A.

Vbc = (1.33 A)(40.0 ) = 53.3 V.

P = (53.3 V)2/(200 ) =      14.2 W.

(c)	For x = 0.25 we have

Req = (0.25)(100 )(200 )/[(0.25)(100 ) + 200 ] = 22.2 .

I = (120 V)/[(1 – 0.25)(100 ) + 22.2 ] = 1.23 A.

Vbc = (1.23 A)(22.2 ) = 27.4 V.

P = (27.4 V)2/(200 ) =      3.76 W.

76.	(a)	When there is no current through the galvanometer,

the current I must pass through the long resistor R, so

the potential difference between A and C is

VAC = IR.

Because there is no current through the measured emf,

for the bottom loop we have

å = IR.

When different emfs are balanced, the current I is the same,

so we have

ås = IRs ,   and   åx = IRx .

When we divide the two equations, we get

ås/åx  = Rs /Rx ,   or   åx = (Rx/Rs)ås .

(b)	Because the resistance is proportional to the length, we have

åx = (Rx/Rs)ås  = (45.8 cm/25.4 cm)(1.0182 V) =      1.836 V.

(c)	If we assume that the current in the slide wire is much greater than the galvanometer current, the

uncertainty in the voltage is

?V = ± IGRG = ± (0.015 mA)(30 ) = ± 0.45 mV.

Because this can occur for each setting and there will be uncertainties in measuring the distances,

the minimum uncertainty is      ± 0.90 mV.

(d)	The advantage of this method is that there is      no effect of the internal resistance,      because

there is no current through the cell.

77.	(a)	Normally there is no dc current in the circuit, so the voltage of the battery is across the

capacitor.  When there is an interruption, the capacitor voltage will decrease exponentially:

VC = V0 e – t/ .

We find the time constant from the need to maintain 70% of the voltage for 0.20 s:

0.70V0 = V0 e – (0.20 s)/ ,   or   (0.20 s)/ = ln(1.43) = 3.57,

which gives  = 0.56 s.

We find the required resistance from

 = RC;

0.56 s = R(1410–6 F), which gives R = 4.0104  =        40 k.

(b)	In normal operation, there will be no voltage across the resistor, so the device should be connected

between b and c.

78.	The charge on the capacitor and the current in the resistor decrease exponentially:

Q = Q0 e – t/ , I = I0 e – t/  , with I0 = Q0 /.

When we express the stored energy in terms of charge we have

U = !CV2 = !Q2/C = !(Q02/C)(e – t/ )2.

Thus the change in stored energy for an elapsed time equal to one time constant is

?Ustored = !(Q02/C)(e – 2/ ) – !(Q02/C) = !(Q02/C)(e – 2 – 1).

Because the current varies, we find the thermal loss by integrating:

79.	To get an output voltage of 100 V, it is necessary to put solar cells in series.  The number of series cells required is

Nseries = (100 V)/(0.80 V/cell) = 125 cells.

To get an output current of 1.0 A, it is necessary to put solar cells in parallel.  The number of parallel cells required is

Nparallel = (1.0 A)/(0.350 A) = 2.8 = 3 cells.

We connect 125 cells in series and then 3 of these units in parallel.  Thus the total number of cells is

N = NseriesNparallel = (125)(3) =      375 cells.

The size of the panel is

(125)(0.030 m)  (3)(0.030 m) =      3.8 m  0.090 m.

Of course it is possible to adjust the dimensions by changing the wiring of the cells.  To optimize the output it is necessary to have the panel move so that it is always perpendicular to the sunlight.

80.	We have labeled the resistors and the currents through the resistors with the value of the resistance.  For the conservation of current at point c, we have

I4 + I5 = I6 + I12 .

For the three loops indicated on the diagram, we have

loop 1:	å10 – I5R5 – I6R6 + å5 = 0;

+ 10.00 V – I5(5.00 ) – I6(6.00 ) + 5.00 V = 0;

15.00 V = I5(5.00 ) + I6(6.00 );

loop 2:	å4 + å8 – I12R12 – I4R4 = 0;

+ 4.00 V + 8.00 V – I2(12.00 ) – I4(4.00 ) = 0;

12.00 V = I4(4.00 ) + I12(12.00 );

loop 3:	I12R12 – I6R6 = 0;

I12(12.00 ) – I6(6.00 ) = 0.

When we solve these equations, we get

I4 = 0.857 A, I5 = 1.296 A, I6 = 1.428 A, I12 = 0.714 A.

We have carried an extra decimal place to show the agreement with the junction equations.

81.	(a)	Immediately after the switch is closed, there is no charge

on the capacitor, so the voltage across C, and thus across R2 ,

is zero.  Thus the current is

I0 = å/R1 = (10.0 V)/(20 ) =       0.50 A.

(b)	After a long time the charge on the capacitor will be maximum,

so there will be no current through the capacitor.

Thus the current is

I = å/(R1 + R2) = (10.0 V)/(20  + 40 ) =       0.17 A.

(c)	For the potential difference Vbc we have

Vbc = IR2 = Qmax/C;

(0.167 A)(40 ) = Qmax/(0.50 F), which gives Qmax =       3.3 C.

(d)	When the switch is opened, the capacitor will discharge through R2 .  The time constant for

this circuit is

 = R2C = (40 )(0.50 F) = 20 s.

The capacitor charge will decrease exponentially:

Q = Qmax e – t/ ;

0.20Qmax = Qmax e – t/(20 s),   or   t/(20 s) = ln(5.0) = 1.61,

which gives t =      32 s.

82.	(a)	The current in the series circuit is

I = V0/(R1 + R2).

The output voltage is the voltage across R2 :

VT = IR2 = V0R2/(R1 + R2);

3.0 V = (12.0 V)R2/(10.0  + R2), which gives R2 =       3.3 .

(b)	We find the equivalent resistance between b and c:

1/Req = (1/R2) + (1/Rload);

1/Req = (1/3.33 ) + (1/7.0 ), which gives Req = 2.26 .

The current in the series circuit is

I = V0/(R1 + Req).

The output voltage is the voltage across Req :

VT = IReq = V0Req/(R1 + Req) = (12.0 V)(2.26 )/(10.0  + 2.26 ) =      2.2 V.

83.	(a)	After a long time there will be a steady state; there

will be no current in the capacitor branch:

I5 = 0;   I1 = I3 ,   and   I2 = I4.

For the two resistor branches we have

Va – Vb = å = I2(R2 + R4);

12.0 V = I2(1.0  + 9.0 ), which gives I2 = 1.20 A;

Va – Vb = å = I1(R1 + R3);

12.0 V = I1(10.0  + 5.0 ), which gives I1 = 0.80 A.

We can find the potential difference across the capacitor from

Vc – Vd 	= (Vc – Vb) – (Vd – Vb) = I4R4 – I3R3

= (1.20 A)(9.0 ) – (0.80 A)(5.0 ) =      + 6.8 V.

The charge on the capacitor is

Q = C(Vc – Vd) = (1.5 F)(6.8 V) =      10.2 C.

(b)	When the switch is opened, we find the equivalent resistance between c and d:

1/Req = 1/(R1 + R2) + 1/(R3 + R4);

1/Req = [1/(10.0  + 1.0 ) + 1/(5.0  + 9.0 )], which gives Req = 6.16 .

The time constant for this circuit is

 = ReqC = (6.16 )(1.5 F) = 9.24 s.

The capacitor charge will decrease exponentially:

Q = Qmax e – t/ ;

0.050Qmax = Qmax e – t/(9.24 s),   or   t/(9.24 s) = ln(20.0) = 3.00,

which gives t =      28 s.

84.	(a)	The time constant of the RC circuit is

 = RC = (33.0 k)(4.00 F) = 132 ms.

During the charging cycle, the charge and the voltage

on the capacitor increases exponentially:

VC = V0 (1 – e – t/ ),

so we find the time to reach 90.0 V from

t1/(132 ms) = ln(10.0) = 2.30,

which gives t1 =      304 ms.

(b)	When the neon bulb starts conducting, the voltage on the capacitor drops quickly to 70.0 V and

then starts charging.  We can find the recharging time by first finding the time for the capacitor

to reach 70.0 V:

t3/(132 ms) = ln(3.33) = 1.20,

which gives t3 = 159 ms.

Thus the time to charge from 70.0 V to 90.0 V is

t4 = t1 – t3 = 304 ms – 159 ms = 145 ms.

The time from when the switch is closed is

t2 = t1 + t4 = 304 ms + 145 ms =      449 ms.

(c)