CHAPTER 29 – Electromagnetic Induction and Faraday’s Law

1.	The average induced emf is

å = – N ?B/?t = – (2)[(+58 Wb) – (– 80 Wb)]/(0.72 s) =      – 3.8102 V.

2.	Because the plane of the coil is perpendicular to the magnetic field, the initial flux through the loop is maximal.  The magnitude of the average induced emf is

å = – ?B/?t = – A ?B/?t = – p(0.13 m)2(0 – 0.90 T)/(0.15 s) =      0.32 V.

3.	As the coil is pushed into the field, the magnetic flux increases

into the page.  To oppose this increase, the flux produced by the

induced current must be out of the page, so the induced current is

counterclockwise.

4.	As the magnet is pushed into the coil, the magnetic flux

increases to the right.  To oppose this increase, the flux

produced by the induced current must be to the left, so

the induced current in the resistor will be

from right to left.

5.	The magnitude of the average induced emf is

å = – ?B/?t = – A ?B/?t = – p(0.036 m)2(0 – 1.3 T)/(0.20 s) =      0.026 V.

6.	We choose up as the positive direction.  The average induced emf is

å = – ?B/?t = – A ?B/?t = – p(0.046 m)2[(– 0.25 T) – (+ 0.63 T)]/(0.15 s) = 3.910–2 V =      39 mV.

7.	(a)	The increasing current in the wire will cause an increasing

field into the page through the loop.    To oppose this

increase, the induced current in the loop will produce a flux

out of the page, so the direction of the induced current will be

counterclockwise.

(b)	The decreasing current in the wire will cause a decreasing

field into the page through the loop.    To oppose this

decrease, the induced current in the loop will produce a flux

into the page, so the direction of the induced current will be

clockwise.

(c)	Because the current is constant, there will be no change in flux,

so the induced current will be

zero.

(d)	The increasing current in the wire will cause an increasing

field into the page through the loop.    To oppose this

increase, the induced current in the loop will produce a flux

out of the page, so the direction of the induced current will be

counterclockwise.

8.	(a)	As the resistance is increased, the current in the outer loop

will decrease.  Thus the flux through the inner loop, which is

out of the page, will decrease.  To oppose this decrease, the

induced current in the inner loop will produce a flux out of the

page, so the direction of the induced current will be

counterclockwise.

(b)	If the small loop is placed to the left, the flux through the

small loop will be into the page and will decrease.  To oppose

this decrease, the induced current in the inner loop will produce

a flux into the page, so the direction of the induced current will be

clockwise.

9.	As the solenoid is pulled away from the loop, the magnetic

flux to the right through the loop decreases.  To oppose this

decrease, the flux produced by the induced current must be to

the right, so the induced current is

counterclockwise      as viewed from the solenoid.

10.	(a)	The average induced emf is

å = – ?B/?t = – A ?B/?t = – p(0.10 m)2[(– 0.45 T) – (+ 0.52 T)]/(0.180 s) =       0.17 V.

(b)	The positive result for the induced emf means the induced field is away from the observer, so the

induced current is      clockwise.

11.	(a)	The magnetic flux through the loop is into the paper and decreasing, because the area is

decreasing.  To oppose this decrease, the induced current in the loop will produce a flux

into the paper, so the direction of the induced current will be      clockwise.

(b)	We choose into the paper as the positive direction.  The average induced emf is

å 	= – ?B/?t = – B ?A/?t = – pB ?(r2)/?t

= – p(0.75 T)[(0.030 m)2 – (0.100 m)2]/(0.50 s) = 4.310–2 V =      43 mV.

(c)	We find the average induced current from

I = å/R = (43 mV)/(2.5 ) =      17 mA.

12.	If  is the angle between the magnetic field and the normal to the plane of the loop, the flux through the loop is

B = BA cos  = BA cos t,

so the induced emf is

å = – dB /dt = BA sin t.

For a sinusoidal variation, the rms value is

årms = BA/v2;

7010–3 V = B(0.15 m)2[2p rad/(4510–3 s)]/v2. which gives B =       3.210–2 T.

13.	As the loop is pulled from the field, the flux through the loop decreases.  We find the induced emf from

å = – ?B/?t = – B ?A/?t = – (0 – 0.40 T)p(0.10 m)2/(10010–3 s) = 0.126 V.

The dissipated energy is

Energy = P ?t = (å2/R) ?t = [(0.126 V)2/(150 )](10010–3 s) =      1.110–5 J.

14.	As the loop is pulled from the field, the flux through the loop

will decrease.  We find the induced emf from

å = – ?B/?t = – B ?A/?t = – B¬ ?x/?t = – B¬ (– v) = B¬v.

Because the inward flux is decreasing, the induced flux will

be into the page, so the induced current is clockwise, given by

I = å/R.

Because this current in the left hand side of the loop is in a

downward magnetic field, there will be a magnetic force to

the left.  To keep the rod moving, there must be an equal

external force to the right, which we find from

F = I¬B = (å/R)¬B = B2¬2v/R = (0.450 T)2(0.350 m)2(3.40 m/s)/(0.230 ) =      0.367 N.

15.	For the resistance of the loop, we have

R = L/A = pD/#pd2 = 4D/d2.

The induced emf is

å = – ?B/?t = – #pD2 ?B/?t;

so the induced current is

I = å/R = – (pDd2/16) ?B/?t.

In the time ?t the amount of charge that will pass a point is

Q 	= I ?t

= – (pDd2/16) ?B = – [p(0.156 m)(2.0510–3 m)2/16(1.6810–8  · m)](0 – 0.550 T) =       4.21 C.

16.	The flux through a loop is

B = [(8.8 s–1)t – (0.51 s–3)t3]10–2 T · m2.

(a)	The induced emf is

å 	= – N dB/dt = – (60)[(8.8 s–1) – (1.53 s–3)t2]10–2 T · m2

= [– 528 + (91.8 s–2)t2]10–2 V =        – 5.3 V+ (0.92 V · s–2)t2.

(b)	At t = 1.0 s we have

å1 = – 5.28 V + (0.918 V · s–2)(1.0 s)2 =      – 4.4 V.

At t = 5.0 s we have

å5 = – 5.28 V + (0.918 V · s–2)(5.0 s)2 =      18 V.

17.	(a)	For the resistance of the loop, we have

R = L/A = (1.6810–8  · m)(20)p(0.350 m)/p(1.010–3 m)2 = 0.118 .

The induced emf is

å = – ?B/?t = – NA ?B/?t = – (20)p(0.175 m)2(3.2010–3 T/s) = – 6.1610–3 V.

Thus the induced current is

I = å/R = (6.1610–3 V)/(0.118 ) =      5.210–2 A.

(b)	Thermal energy is produced in the wire at the rate of

P = I 2R = (5.210–2 A)2(0.118 ) = 3.210–4 W =       0.32 mW.

18.	Inside the solenoid, the magnetic field is B = 0nI, so the flux through the loop is

B = A1B = A10nI0 cos t.

The induced emf is

å = – dB/dt =       A10nI0 sin t.

19.	The flux through the loop is

B = BA.

The induced emf is

å = – dB/dt = – B dA/dt = – (0.48 T)(– 3.5010–2 m2/s) =       1.710–2 V.

Because the area changes at a constant rate, this is the induced emf for both times.

20.	The flux through the loop when it has area A is

B = BA = Bpr2.

The induced emf is

å = – dB/dt = – B2pr dr/dt.

At t = 0  the radius is

r0 = (A0/p)1/2 = (0.285 m2/p)1/2 = 0.301 m.

At a later time t the radius is

r = r0 + (dr/dt)t,

so the induced emf is

å = – 2pB[r0 + (dr/dt)t] dr/dt.

At t = 0 we have

å0 = – 2p(0.48 T)[0.301 m + (0.0700 m/s)(0)](0.0700 m/s) =      – 0.064 V.

At t = 1.00 s we have

å1 = – 2p(0.48 T)[0.301 m + (0.0700 m/s)(1.00 s)](0.0700 m/s) =      – 0.078 V.

21.	At a distance r from the wire, the magnetic field is directed into

the paper with magnitude

B = 0I/2pr.

Because the field is not constant over the square, we find the

magnetic flux by integration.  We choose a differential element

parallel to the wire at position r with area a dr.  The magnetic

field through this element is constant, so the flux is

22.	Because the velocity is perpendicular to the magnetic

field and the rod, we find the induced emf from

å 	= B¬v

= (0.750 T)(0.190 m)(0.250 m/s)

= 3.5610–2 V =      35.6 mV.

23.	(a)	Because the velocity is perpendicular to the magnetic

field and the rod, we find the induced emf from

å 	= B¬v

= (0.35 T)(0.240 m)(1.8 m/s) =      0.15 V.

(b)	We find the induced current from

I = å/R = (0.151 V)/(2.2  + 26.0 ) =      5.410–3 A.

(c)	The induced current in the rod will be down.  Because this

current is in an outward magnetic field, there will be a

magnetic force to the left.  To keep the rod moving, there

must be an equal external force to the right, which we find from

F = I¬B = (5.410–3 A)(0.240 m)(0.35 T) =      4.510–4 N.

24.	If we assume that the movable rod starts at the bottom of the U, in a time t it will have moved a distance x = vt.  For the resistance of the U, we have

R = L/A = (2vt + ¬)/A.

The induced emf is

å = B¬v;

so the induced current is

I = å/R = B¬v/[(2vt + ¬)/A] =      B¬vA/(2vt + ¬).

25.	(a)	Although there is an induced emf, there is no current

because there is no closed circuit.  Thus the rod

will move at constant speed.

(b)	When the circuit is completed, there will be a current.

The induced emf is B¬v.  Because the only resistance is

from the rod, the current is

I = B¬v/R.

The induced current in the rod will be down.  Because this

current is in an outward magnetic field, there will be a

magnetic force to the left, which will produce an acceleration:

F = – IB¬ = – B2¬2v/R = ma = m dv/dt, or – (B2¬2/mR) dt = dv/v.

We integrate to get the speed:

The speed eventually goes to zero.  This is an application of Lenz’s law.

26.	(a)	If the polarity of the source of emf is as indicated

in the figure, the current in the rod will be up, which

will produce a force in the outward magnetic field

that is to the right.  For a constant current, the

constant force will produce a constant acceleration:

F = I¬B = ma.

Thus the speed of the rod is

v = v0 + at = 0 + (I¬B/m)t =       I¬Bt/m.

Note that there will be an induced emf in the rod, so

the source must compensate to maintain the constant current.

(b)	With a constant source emf å0 , the current will be determined by the two emfs in the circuit.

Because the initial motion is to the right, the induced emf will be down, opposing the source.

Thus we have

I = (å0 – B¬v)/R.

The force produced by this current provides the acceleration:

F = I¬B = (å0 – B¬v)B¬/R = m dv/dt,  or  (B¬/mR) dt = dv/(å0 – B¬v).

When we integrate, we get

(c)	When the current is constant, we see that the speed continues to increase, so no terminal speed is

reached.  With a constant å0 , as t  8, the exponential term goes to 0, so there is a terminal speed:

vterm = å0/B¬.

27.	(a)	At a distance r from the long wire, the magnetic field is

directed into the paper with magnitude

B = 0I/2pr.

Because the field is not constant over the short section, we

find the induced emf by integration.  We choose a differential

element dr a distance r from the long wire.

The induced emf in this element is

då = Bv dr toward the long wire.

We find the total emf by integrating:

å =

(b)	If the current is in the opposite direction to I, the only change will be in the direction of the emf:

å =

28.	The maximum induced emf is

å = NBA.

If the only change is in the rotation speed, for the two conditions we have

å2/å1 = 2/1 ;

å2/(12.4 V) = (2500 rpm)/(950 rpm) which gives å2 =      32.6 V.

29.	The induced emf is

å = NBA sin t.

For the rms value of the output, we have

Vrms = [(å2)av]1/2 = [(NBA)2(sin2 t)av]1/2 = NBA [(sin2 t)av]1/2.

The sin2 t function varies from 0 to 1, with an average value of 1/2, so we get

Vrms = NBA(1/v2) = NBA/v2.

30.	We find the rotation speed from

åpeak = NBA;

120 V = (420 turns)(0.350 T)(0.210 m)2, which gives  = 18.5 rad/s =       2.95 rev/s.

31.	We find the peak emf from

åpeak = NBA = (350 turns)(0.45 T)p(0.050 m)2(60 rev/s)(2p rad/rev) = 466 V.

The rms voltage output is

Vrms = åpeak/v2 = (466 V)/v2 = 330 V =      0.33 kV.

If only the rotation frequency changes, to double the rms voltage output, we must double the rotation speed, so we have

f2 = 2f1 = 2(60 rev/s) =       120 rev/s.

32.	We find the counter emf from

å – åback = IR;

120 V – åback = (9.20 A)(3.75 ), which gives åback =      86 V.

33.	Because the counter emf is proportional to the rotation speed, for the two conditions we have

åback2/åback1 = 2/1 ;

åback2/(72 V) = (2500 rpm)/(1800 rpm), which gives åback2 =      100 V.

34.	Because the counter emf is proportional to both the rotation speed and the magnetic field, for the two conditions we have

åback2/åback1 = (2/1)(B2/B1);

(75 V)/(100 V) = [(2500 rpm)/(1000 rpm)](B2/B1), which gives       B2/B1 = 0.30.

35.	Because the counter emf is proportional to the rotation speed, we find the new value from

åback2/åback1 = 2/1 ;

åback2/(108 V) = (1/2), which gives åback2 = 54 V.

We find the new current from

å – åback = IR;

120 V – 54 V = I(5.0 ), which gives I =      13 A.

36.	(a)	We find the emf of the generator from the load conditions:

V = å – IarmatureRarmature;

200 V = å – (50 A)(0.40 ), which gives å = 220 V.

When there is no load on the generator, the current is zero, so the voltage will be the emf:       220 V.

Note that no-load means little torque required to turn the generator.

(b)	Because the generator emf is proportional to the rotation speed, we find the new value from

å2/å1 = 2/1 ;

å2/(220 V) = (800 rpm/1000 rpm), which gives å2 = 176 V.

We find the new load voltage from

V2 = å2 – IarmatureRarmature  = 176 V – (50 A)(0.40 ) =      156 V.

Note that the power output is reduced to 7.8 kW.

37.	We find the number of turns in the secondary from

VS/VP = NS/NP ;

(8500V)/(120 V) = NS/(500 turns), which gives NS =       3.54104 turns.

38.	Because NS < NP , this is a      step-down     transformer.

We find the ratio of the voltages from

VS/VP = NS/NP = (120 turns)/(720 turns) =      0.167.

With 100% efficiency, for the ratio of currents we have

IS/IP = NP/NS = (720 turns)/(120 turns) =     6.00.

39.	With 100% efficiency, the power on each side of the transformer is the same:

IPVP = ISVS , so we have

IS/IP = VP/VS = (22 V)/(120) =      0.18.

40.	We find the ratio of the number of turns from

NS/NP = VS/VP = (12103 V)/(220 V) =      55.

If the transformer is connected backward, the role of the turns will be reversed:

VS/VP = NS/NP ;

VS/(220 V) = 1/55, which gives VS =       4.0 V.

41.	(a)	We assume 100% efficiency, so we have

IPVP = ISVS ;

(0.65 A)(120 V) = (15 A)VS , which gives VS =       5.2 V.

(b)	Because VS < VP , this is a      step-down     transformer.

42.	(a)	We assume 100% efficiency, so we find the input voltage from

P= IPVP ;

100 W = (26 A)VP , which gives VP = 3.8 V.

Because VS > VP , this is a      step-up     transformer.

(b)	For the voltage ratio we have

VS/VP = (12 V)/(3.84 V) =      3.1.

43.	We find the output voltage of this step-up transformer from

VS/VP = NS/NP ;

VS/(120 V) = (1510 turns)/(330 turns), which gives VS =       549 V.

We find the input current from

IS/IP = NP/NS ;

(15.0 A)/IP = (330 turns)/(1510 turns), which gives IP =     68.6 A.

44.	(a)	We can find the current in the transmission lines from the output emf:

Pout = IVout ;

30106 W = I(45103 V), which gives I = 667 A.

We find the input emf from

åin = Vout + IRlines = 45103 V + (667 A)(3.0 ) = 47103  =      47 kV.

(b)	The power loss in the lines is

Ploss = I 2Rlines = (667 A)2(3.0 ) = 1.33106 W = 1.33 MW.

The total power is

Ptotal = Pout + Ploss = 30 MW + 1.33 MW = 31.3 MW,

so the fraction lost is

(1.33 MW)/(31.3 MW) =     0.042 (4.2%).

45.	Without the transformers, we can find the delivered current, which is the current in the transmission lines, from the delivered power:

Pout = IVout ;

65103 W = I(120V), which gives I = 542 A.

The power loss in the lines is

PL0 = I 2Rlines = (542 A)2(2)(0.100 ) = 5.87104 W = 58.7 kW.

With the transformers, to deliver the same power at 120 V, the delivered current from the step-down transformer is still 542 A.

If the step-down transformer is 99% efficient, we have

(0.99)IP2VP2 = IS2VS2 ;

(0.99)IP2(1200 V) = (542 A)(120 V), which gives IP2 = 54.7 A.

Because this is the current in the lines, the power loss in the lines is

PL2 = IP22Rlines = (54.7 A)2(2)(0.100 ) = 5.99102 W = 0.599 kW.

For the 1% losses in the transformers, we approximate the power in each transformer:

PLt = (0.01)(65 kW + 65.6 kW) = 1.31 kW.

The total power loss using the transformers is

PL = 0.599 kW + 1.31 kW = 1.9 kW.

The power saved by using the transformers is

Psaved = PL0 – PL = 58.7 kW – 1.9 kW =       56.8 kW.

46.	We can find the delivered current, which is the current in the transmission lines, from the delivered power:

Pout = IVout ;

300106 W = I(600103 V), which gives I = 500 A.

The 2% loss means Pout = 0.98 Pin .   Thus the power loss in the lines is

PL = (0.02)Pin = (0.02)Pout /(0.98)= I 2Rlines ;

(0.02)(300106 W)/(0.98) = (500 A)2Rlines , which gives Rlines = 24.5 .

We find the radius of the 2 lines, each 200 km long, from

R = L/A ;

24.5  = (2.6510–8  · m)(2)(200103 m)/pr2 , which gives r = 1.17410–2 m.

Thus the diameter of the lines should be      2.35 cm.

47.	We find the electric field from

åinduced = B¬v = ý E · d¬ = E¬, which gives

E = Bv = (0.750 T)(0.250 m/s) =       0.188 V/m.

48.	The induced emf around a circle is

å = – dB /dt = – A dB/dt.

Because A and dB/dt are the same for the two circular

paths, å is the same.  Even though E is greater in the

region of the outer circle, the integral ý E · d¬ is the

same.  Note that for part of the path the component

of E is parallel to d¬ and for part it is antiparallel.

49.	(a)	The changing magnetic field through the area defined by the circular vacuum tube produces a

circular electric field along the axis of the tube.  This electric field along the electron path creates

the acceleration to increase the speed of the electron.

(b)	The magnetic force on the electron provides the centripetal acceleration.  To get a force toward the

center, the current must be in on the right and out on the left, that is, counterclockwise.  Because

electrons have a negative charge, they must be moving       clockwise.

(c)	The electric field along the electron path is given by the rate of change of the magnetic flux:

ý E · d¬ = – dB/dt = – A dB/dt.

If we choose clockwise (the direction of the electron motion) for the positive direction, the electric

field must be negative, so        dB/dt > 0.

(d)	To provide the radial acceleration, the magnetic field

must be down (positive).  To provide the tangential

acceleration, the magnetic field must be increasing.

In a sinusoidal variation, these two conditions exist

only for # of the cycle.

50.	We find the electric field along the electron’s path at a radius R by applying Faraday’s law:

ý E · d¬ = – dB/dt = – A dBav/dt;

E2pR = – pR2 dBav/dt,  or  E = – !R dBav/dt.

Thus for the tangential acceleration we have

atan = dv/dt = – eE/m = !(eR/m) dBav/dt.

The magnetic force at the position of the electron orbit provides the centripetal acceleration:

evB0 = mv2/R, which gives v = eB0R/m.

If we differentiate this we get

dv/dt = (eR/m) dB0/dt.

When we compare this to the previous result, we see that

dB0/dt = ! dBav/dt, at any moment.

This is satisfied at any time if B0 = !Bav .

51.	(a)	Because the current in the rod is constant, the potential difference across the rod must be

constant: V = IR.  The net electric field is the potential gradient along the rod:

Enet = V/¬ =       IR/¬ (constant).

(b)	With a constant source emf å0 , the potential difference across the rod will be

V = IR = å0 – B¬v.

If we use the result for the velocity from Problem 26, we have

The net electric field is the potential gradient along the rod:

52.	(a)	The clockwise current in the left-hand loop produces a magnetic field which is into the page

within the loop and out of the page outside the loop.  Thus the right-hand loop is in a magnetic

field out of the page.  Before the current in the left-hand loop reaches its steady state, there will

be an induced current in the right-hand loop that will produce a magnetic field into the page to

oppose the increase of the field from the left-hand loop.  Thus the induced current will be

clockwise.

(b)	After a long time, the current in the left-hand loop will be constant, so there will be

no induced current.

(c)	If the second loop is pulled to the right, the magnetic field out of the page from the left-hand loop

through the second loop will decrease.  During the motion, there will be an induced current in the

right-hand loop that will produce a magnetic field out of the page to oppose the decrease of the

field from the left-hand loop.  Thus the induced current will be     counterclockwise.

53.	We find the number of turns from

åpeak = NBA;

24.0 V = N(0.420 T)(0.070 m)2(60 rev/s)(2p rad/rev), which gives N =      31 turns.

54.	The average induced emf is

å = – ?B/?t = – A ?B/?t = – (0.240 m)2(0 – 0.755 T)/(0.0400 s) = 1.087 V.

The average current is

I = å/R = (1.087 V)/(6.50 ) = 0.167 A.

The energy dissipated is

Energy = I2R ?t = (0.167 A)2(6.50 )(0.0400 s) =      7.2710–3 J.

55.	A side view of the rail and bar is shown in the figure.  The

component of the velocity of the bar that is perpendicular to

the magnetic field is v cos , so the induced emf is

å = B¬v cos .

If we assume the resistance of the copper bar can be neglected,

this produces a current in the bar

I = å/R = (B¬v cos )/R into the page.

Because the current is perpendicular to the magnetic field, the

force on the bar from the magnetic field will be horizontal,

as shown, with magnitude

FB = I¬B = (B2¬2v cos )/R.

For the bar to slide down at a steady speed, the net force must be zero.  If we consider the components along the rail, we have

FB cos  – mg sin  = 0,   or

[(B2¬2v cos )/R] cos  = (B2¬2v cos2 )/R = mg sin 

(0.55 T)2(0.30 m)2v(cos2 5.0°)/(0.60 ) = (0.040 kg)(9.80 m/s2) sin 5.0°, which gives      v = 0.76 m/s.

56.	(a)	The voltage drop across the lines is

?V = 2IR = 2(700 A)(0.80 ) = 1.12103 V = 1.12 kV.

Thus the voltage at the other end is

Vout = Vin – ?V = 42 kV – 1.12 kV =       41 kV.

(b)	The power input is

Pin = IVin = (0.700 kA)(42 kV) =       29.4 MW.

(c)	The power loss in the lines is

Ploss = 2I 2R = 2(0.700 kA)2(0.80 ) =       0.78 MW.

(d)	The power output is

Pout = IVout = (0.700 kA)(40.9 kV) =       28.6 MW.

Note that this is Pin – Ploss .

57.	If we assume perfect transformers, the only losses will be the energy dissipated in the transmission lines.  The resistance of the lines is

R = (0.10 /km)(100 km) = 10 .

If Pin is the power supplied to the town, we have

Pin/(Pin + Iline2R) = 0.985;

(50106 W)/[50106 W + Iline2(10 )] = 0.985, which gives Iline = 276 A.

For the perfect transformer at the town end of the line, we have

Pin = IlineV2 ;

50103 kW = (276 A)V2 , which gives V2 = 181 kV.

The voltage drop in the transmission line is

Vdrop = IlineR = (0.276 kA)(10 ) = 2.76 kV.

Thus the voltage at the station must be stepped up to

V1 = V2 + Vdrop = 181 kV + 2.76 kV =      184 kV.

58.	The magnetic field through the loop is B = 0nI inside the

solenoid only, so the flux through the loop is

B = NBA = N0nIprsolenoid2.

The current induced in the loop is

I 	= å/R = – (1/R) dB /dt = – (N0nprsolenoid2/R) dI/dt

= – [(150 turns)(4p10–7 T · m/A)(200102 turns/m)p(0.045 m)2/(11.0 )][(2.0 A)/(0.10 s)]

=      – 4.410–2 A (opposite to the direction of the current in the solenoid).

This assumes the windings of the coil and the solenoid are in the same direction.

59.	The magnitude of the average induced emf is

å = ?B/?t,

so the average current is

I = å/R = (1/R) ?B/?t.

The charge moving past a fixed point is

q = I ?t = ?B/R = (B/R) ?A.

The number of electrons is

N = q/e = (B/eR) ?A.

The maximum number of electrons occurs for the maximum change in area in a 90° rotation.  The area perpendicular to the field varies sinusoidally.  From the plot we see that the greatest change occurs when the coil moves from an angle of 45° with the field to an angle of 45° on the other side.  Thus we have

Nmax = [(0.15 T)p(0.030 m)2/(1.6010–19 C)(0.025 )][cos 45° – (– cos 45°)] =      1.51017.

60.	We find the peak emf from

åpeak = NBA = (125 turns)(0.200 T)(0.0660 m)2(120 rev/s)(2p rad/rev) =      82.1 V.

61.	(a)	Because we have direct coupling, the torque provided by the motor balances the torque of the

friction force:

NIAB = Fr;

(300 turns)I(0.10 m)(0.15 m)(0.60 T) = (250 N)(0.25 m), which gives I =       23 A.

(b)	To maintain the speed, we have a force equal to the friction force, so the power required is

Fv = (250 N)(30 km/h)/(3.6 ks/h) = 2.08103 W.

This must be provided by the net power from the motor, which is

Pnet = IVin – I 2R = I(Vin – IR) = Iåback = Fv;

(23 A)åback = 2.08103 W, which gives åback =      90 V.

(c)	The power dissipation in the coils is

Ploss = Pin – Pnet = (23 A)(10)(12 V) – 2.08103 W =       6.9102 W.

(d)	The useful power percentage is

(Pnet/Pin)(100) = (Iåback/IVin)(100) = (90 V/120 V)(100) =      75%.

62.	The average induced emf is

å = – ?B/?t = – NA ?B/?t = – NA[(– B) – (+ B)]/?t = 2NAB/?t.

The average current is

I = å/R = 2NAB/R ?t,

so the total charge that passes through the galvanometer is

Q = I ?t = (2NAB/R ?t) ?t = 2NAB/R,   or   B = QR/2NA.

63.	(a)	From the efficiency of the transformer, we have PS = 0.80PP.

For the power input to the transformer, we have

PP = IPVP ;

(75 W)/0.80 = IP(110 V), which gives IP =       0.85 A.

(b)	We find the secondary voltage from

PS = VS2/RS ;

75 W = VS2/(2.4 ), which gives VS = 13.4 V.

We find the ratio of the number of turns from

NP/NS = VP/VS = (110 V)/(13.4 V) =      8.2.

64.	The induced emf is

å = – dB/dt = – Npr2 dB/dt = (Npr2B0/)e – t/.

Thus the rate at which energy is dissipated is

P = I2R = å2/R = [(Npr2B0/)2/R]e – 2t/.

Because this rate is a function of time, we find the energy dissipated by integrating:

65.	We choose a differential element dr a distance r from the

center of rotation.  The speed of this element is v = r, so

the differential induced emf is

då = Bv dr = Br dr.

We integrate to find the total emf:

å =

66.	(a)	Because VS < VP , this is a      step-down     transformer.

(b)	We assume 100% efficiency, so we find the current in the secondary from

P= ISVS ;

40 W = IS(12 V), which gives IS =       3.3 A.

(c)	We find the current in the primary from

P= IPVP ;

40 W = IP(120 V), which gives IP =       0.33 A.

(d)	We find the resistance of the bulb from

VS = ISRS ;

12 V = (3.33 A)RS , which gives RS =       3.6 .

67.	We can find the current in the transmission lines from the power transmitted to the user:

PT = IV,   or   I = PT/V.

The power loss in the lines is

PL = I 2RL = (PT/V)2RL = (PT)2RL/V2.

68.	(b)	At start up there will be no induced emf in the 		(a)

armature.  Because the line voltage is across each

resistor, we find the currents from

Ifield0 = å/Rfield = (115 V)/(36.0 ) = 3.19 A;

Iarmature0 = å/Rarmature = (115 V)/(3.00 ) = 38.3 A.

We use the junction condition to find the total current:

I0 = Ifield0 + Iarmature0 = 3.19 A + 38.3 A =       41.5 A.

(c)	At full speed, the back emf is maximum.  Because the

line voltage is across the field resistor, we find the

field current from

Ifield = å/Rfield = (115 V)/(36.0 ) = 3.19 A.

We find the armature current from

å – åback = IarmatureRarmature ;

115 V – 105 V = Iarmature(3.00 ),

which gives Iarmature = 3.33 A.

Thus the total current is

I0 = Ifield + Iarmature = 3.19 A + 3.33 A =       6.52 A.

69.	If we apply Faraday’s law to a counterclockwise path around the square,

we have

ý E · d¬ = – dB/dt = 0,

because the electric field is static.  We can express the integral as

ý E · d¬ = ?bottom E · d¬ + ?sides E · d¬ + ?top E · d¬ = 0.

On the bottom segment E and d¬ are parallel, so ?bottom E · d¬ > 0.

If there were no fringing,

?sides E · d¬ = 0 because E and d¬ are perpendicular, and

?top E · d¬ = 0 because E = 0.

However, we must have

?sides E · d¬ + ?top E · d¬ < 0, so there must be a fringing field outside

the edge of the plates.  We see that on the side and the top segments there

will be a component of E antiparallel to d¬.

70.	We find the flux through the area swept out by a radial line

on the disk:

B = B(/2p)pR2 = !BtR2.

The magnitude of the induced emf is

å = dB/dt =       !BR2.

The magnetic flux through the area is increasing out of

the page, so the induced flux will be in, which means the

induced emf along the radial line is radially out from

the axis, and the rim is at the higher potential.

71.	We choose a differential element dr a distance r from the

center of rotation.  The speed of this element is v = r, so

the radial differential induced emf is

då = Bv dr = Br dr.

The electric field is the potential gradient:

E = då/dr =       Br, radially out from the axis.

72.	We approximate the small area as a square with side D.

Because the area A = D2 is small, the speed of the area is

v = ¬.  Thus the induced radial emf is

å = BDv = BD¬.

This emf produces a radial current in the area.  If we

neglect the resistance of the outside return path of the

current, the approximate resistance is

R = D/Dd = /d.

The current is approximately

I = å/R = BD¬d/.

This current in the magnetic field produces a tangential

force and thus a torque about the axis:

 = F¬ = IBD¬ = B2D2¬2d/ =       B2A¬2d/.