CHAPTER 31 – AC Circuits

1.	(a)	The reactance of the capacitor is

XC1 = 1/2pf1C = 1/2p(60 Hz)(7.210–6 F) =       3.7102 .

(b)	For the new frequency we have

XC2 = 1/2pf2C = 1/2p(1.0106 Hz)(7.210–6 F) =       2.210–2 .

2.	We find the frequency from

XL = L = 2pfL;

660  = 2pf(22.010–3 H), which gives f = 4.77103 Hz =      4.77 kHz.

3.	We find the frequency from

XC = 1/2pfC;

6.70103  = 1/2pf(2.4010–6 F), which gives f =       9.90 Hz.

4.	The impedance is Z = XC = 1/2pfC.

5.	The impedance is Z = XL = L = 2pfL.

6.	We find the impedance from

Z = XL = L = 2pfL = 2p(33.3 kHz)(36.010–3 H) =       7.53 k.

For the rms current we have

Irms = Vrms/XL = (750 V)/(7.53 k) =      99.6 mA.

7.	If there is no current in the secondary, there will be no induced emf from the mutual inductance.  We find the impedance from

Z = XL = Vrms/Irms = (110 V)/(2.2 A) = 50 .

We find the inductance from

XL = 2pfL;

50  = 2p(60 Hz)L, which gives L =      0.13 H.

8.	(a)	We find the impedance from

Z = XC = 1/2pfC = 1/2p(600 Hz)(0.03610–6 F) = 7.37103  =      7.4 k.

(b)	We find the peak value of the current from

Ipeak = v2 Irms = v2(Vrms /Z) = v2(22 kV)/(7.37 k) =      4.2 A.

The frequency of the current will be the frequency of the line:      600 Hz.

9.	(a)	If the voltage is

V = V0 sin t,

the charge on the capacitor is

Q = CV = CV0 sin t.

Thus the current is

I = dQ/dt = CV0 cos t = CV0 sin (t + 90°).

(b)	If the voltage is

V = V0 sin t,

for the circuit we have

V = V0 sin t = L dI/dt,  or dI/dt = (V0 /L) sin t.

When we integrate this we get

I = – (V0/L) cos t = (V0/L) sin (t – 90°).

10.	The average power dissipation is

P = Irms2R = !Ipeak2R = !(1.80 A)2(260 ) =      421 W.

11.	Because the capacitor and resistor are in parallel, their currents are

IC = V/XC ,   and  IR = V/R.

The total current is I = IC + IR .

(a)	The reactance of the capacitor is

XC1 = 1/2pf1C = 1/2p(60 Hz)(0.3510–6 F) = 7.58103  = 7.58 k.

For the fraction of current that passes through C, we have

fraction1 	= IC1/(IC1 + IR) = (1/XC1)/[(1/XC1) + (1/R)] = R/(R + XC1)

= (0.400 k)/(0.400 k + 7.58 k) = 0.050 =      5.0%.

(b)	The reactance of the capacitor is

XC2 = 1/2pf2C = 1/2p(60,000 Hz)(0.3510–6 F) = 7.58 .

For the fraction of current that passes through C, we have

fraction2 	= IC2/(IC2 + IR) = (1/XC2)/[(1/XC2) + (1/R)] = R/(R + XC2)

= (400 )/(400  + 7.58 ) = 0.98 =      98%.

Thus most of the high-frequency current passes through the capacitor.

12.	(a)	The reactance of the capacitor is

XC1 = 1/2pf1C = 1/2p(60 Hz)(6.810–6 F) = 390  = 0.39 k.

The impedance of the circuit is

Z1 = (R2 + XC12)1/2 = [(1.20 k)2 + (0.39 k)2]1/2 =       1.26 k.

(b)	The reactance of the capacitor is

XC2 = 1/2pf2C = 1/2p(60,000 Hz)(6.810–6 F) = 0.39  = 0.00039 k.

The impedance of the circuit is

Z2 = (R2 + XC22)1/2 = [(1.20 k)2 + (0.00039 k)2]1/2 =       1.20 k.

13.	(a)	The reactance of the inductor is

XL1 = 2pf1L = 2p(50 Hz)(26.010–3 H) = 8.17 .

The impedance of the circuit is

Z1 = (R2 + XL12)1/2 = [(9.0 k)2 + (0.00817 k)2]1/2 =       9.0 k.

(b)	The reactance of the inductor is

XL2 = 2pf2L = 2p(30 kHz)(26.010–3 H) = 4.9 k.

The impedance of the circuit is

Z2 = (R2 + XL22)1/2 = [(9.0 k)2 + (4.9 k)2]1/2 =       10.2 k.

14.	We find the impedance from

Z = Vrms/Irms = (120 V)/(70 mA) =      1.7 k.

15.	(a)	The reactance of the capacitor is

XC = 1/2pfC = 1/2p(60 Hz)(0.8010–6 F) = 3.32103  = 3.32 k.

The impedance of the circuit is

Z = (R2 + XC2)1/2 = [(6.0 k)2 + (3.32 k)2]1/2 = 6.86 k.

The rms current is

Irms = Vrms/Z = (120 V)/(6.86 k) =      18 mA.

(b)	We find the phase angle from

cos  = R/Z = (6.0 k)/(6.86 k) = 0.875.

In an RC circuit, the current leads the voltage, so  =      – 29°.

(c)	The power dissipated is

P = Irms2R = (17.510–3 A)2(6.0103 ) =        1.8 W.

(d)	The rms readings across the elements are

VR = IrmsR = (17.5 mA)(6.0 k) =      105 V;

VC = IrmsXC = (17.5 mA)(3.32 k) =      58 V.

Note that, because the maximal voltages occur at different times, the two readings do not add

to the applied voltage of 120 V.

16.	(a)	The reactance of the inductor is

XL = 2pfL = 2p(60 Hz)(0.250 H) = 94.2 .

The impedance of the circuit is

Z = (R2 + XL2)1/2 = [(765 )2 + (94.2 )2]1/2 = 770.8 .

The rms current is

Irms = Vrms/Z = (120 V)/(770.8 ) =      0.156 A.

(b)	We find the phase angle from

cos  = R/Z = (765 )/(770.8 ) = 0.993.

In an RL circuit, the current lags the voltage, so  =      + 7.02°.

(c)	The power dissipated is

P = Irms2R = (0.156 A)2(765 ) =        18.5 W.

(d)	The rms readings across the elements are

VR = IrmsR = (0.156 A)(765 ) =      119 V;

VL = IrmsXL = (0.156 A)(94.2 ) =      15 V.

Note that, because the maximal voltages occur at different times, the two readings do not add

to the applied voltage of 120 V.

17.	The reactances and impedance in the circuit are

XL = 2pfL = 2p(60 Hz)(3510–3 H) = 13.2 .

XC = 1/2pfC = 1/2p(60 Hz)(2010–6 F) = 133 .

Z = [R2 + (XL – XC)2]1/2 = [(2.0 )2 + (13.2  – 133 )2]1/2 = 119 .

(a)	The rms current is

Irms = Vrms/Z = (45 V)/(119 ) =      0.38 A.

(b)	We find the phase angle from

cos  = R/Z = (2.0 )/(119 ) = 0.0168.

Because XC > XL , the current leads the voltage, so  =      – 89°.

(c)	The power dissipated is

P = Irms2R = (0.38 A)2(2.0 ) =        0.29 W.

18.	For the current and voltage to be in phase, the net reactance of the capacitor and inductor must be zero.  Thus we have

XL = XC;

2pfL = 1/2pfC,  or

f = (1/2p)(1/LC)1/2;

360 Hz = (1/2p)[1/(4010–3 H)C]1/2, which gives C = 4.910–6 F =      4.9 F.

19.	We find the resistance from

Z = [R2 + XL2]1/2 ;

335  = [R2 + (45.5 )2]1/2, which gives R =       332 .

20.	The power dissipated is the power delivered to the circuit:

P = IV = I0V0 sin t sin (t + ).

If we expand the second term, we get

P 	= I0V0 sin t (sin t cos  + cos t sin )

= I0V0 (cos  sin2 t + sin  sin t cos t ) = I0V0(cos  sin2 t + sin  ! sin 2t ).

When we average over a cycle, the average of a sin2 function = !, and the average of a sine function = 0.

Thus we have

= !I0V0 cos .

21.	(a)	The average value over a cycle is

(b)	We assume the half-cycle is from zero to !T:

When we compare this to Vrms , we get

22.	We find the rms current from

P = Irms2R;

15.5 W = Irms2(150 ), which gives Irms = 0.321 A.

We find the impedance from

Z = Vrms/Irms = (60.0 V)/(0.321 A) = 187 .

From this we can find the reactance of the coil:

Z2 = R2 + XL2;

(187 )2 = (150 )2 + XL2, which gives XL = 112 .

We find the frequency that will produce this reactance from

XL = 2pfL ;

112  = 2pf(0.040 H), which gives f =       446 Hz.

23.	The reactance of the capacitor is

XC = 1/2pfC = 1/2p(10.0103 Hz)(500010–12 F) = 3.18103  = 3.18 k.

The reactance of the inductor is

XL = 2pfL = 2p(10.0 kHz)(0.0320 H) = 2.01 k.

The impedance of the circuit is

Z = [R2 + (XL – XC)2]1/2 = [(8.70 k)2 + (2.01 k – 3.18 k)2]1/2 =        8.78 k.

We find the phase angle from

tan  = (XL – XC)/R = (2.01 k – 3.18 k)/(8.70 k) = – 0.134, so  =       – 7.66°.

The rms current is

Irms = Vrms/Z = (800 V)/(8.78 k) =      91.1 mA.

24.	The resonant frequency is

f0 = (1/2p)(1/LC)1/2 = (1/2p)[1/(26.010–6 H)(320010–12 F)]1/2 =       5.52105 Hz.

25.	The resonant frequency is

f0 = (1/2p)(1/LC)1/2 = (1/2p)[1/(30.010–3 H)(12.010–6 F)]1/2 =       265 Hz.

At the resonant frequency, Z = R, so the rate at which energy is supplied by the generator is

P = Irms2R = Vrms2/R = (90.0 V)2/(25.0 ) =      324 W.

26.	(a)	We find the capacitance from the resonant frequency:

f0 = (1/2p)(1/LC)1/2;

33.0103 Hz = (1/2p)[1/(4.1510–3 H)C]1/2, which gives C = 5.6010–9 F =       5.60 nF.

(b)	At resonance the impedance is the resistance, so the current is

I0 = V0/R = (136 V)/(220 ) =      0.618A.

27.	We find the capacitance from the new resonant frequency:

f0 = (1/2p)(1/LC)1/2;

2(33.0103 Hz) = (1/2p)[1/(4.1510–3 H)C]1/2, which gives C = 1.4010–9 F.

At the applied frequency the reactances are

XL = 2pfL = 2p(33.0103 Hz)(4.1510–3 H) = 860  = 0.860 k.

XC = 1/2pfC = 1/2p(33.0103 Hz)(1.4010–9 F) = 3.44103  = 3.44 k.

The impedance is

Z = [R2 + (XL – XC)2]1/2 = [(0.220 k)2 + (0.860 k – 3.44 k)2]1/2 = 2.59 k.

The peak current is

I0 = V0/Z = (136 V)/(2.59 k) =      52.5 mA.

28.	Because the circuit is in resonance, we find the inductance from

f0 = (1/2p)(1/LC)1/2;

18.0103 Hz = (1/2p)[1/L(22010–6 F)]1/2, which gives L = 3.5510–7 H.

If r is the radius of the coil, the number of turns is

N = ¬wire/2pr.

If d is the diameter of the wire, for closely-wound turns, the length of the coil is

¬ = Nd.

Thus the inductance of the coil is

L = 0AN2/¬ = 0pr2(¬wire/2pr)2/Nd = 0¬wire2/4pNd;

3.5510–7 H = (4p10–7 T · m/A)(12.0 m)2/4pN(1.110–3 m), which gives N =      3.68104 turns.

29.	(a)	We can express the charge amplitude in terms of the impedance:

Q0 	= CVC0 = CXCI0 = (1/)(V0/Z)

= V0/[R2 + (L – 1/C)2]1/2 = V0/[(R)2 + (2L – 1/C)2]1/2.

(b)	We find the frequency to make Q0 a maximum by setting the first derivative equal to zero:

dQ0/d = V0(– !)[2R2 + 2(2L – 1/C)(2L)]/[(R)2 + (2L – 1/C)2]3/2 = 0,   or

R2 + 2L(2L – 1/C) = 0, which gives R2/2L = (1/C) – 2L,  or      2 = [(1/LC) – (R2/2L2)].

(c)	For the forced harmonic oscillator, the amplitude is

A0 = F0/m[(2 – 02)2 + b22/m2]1/2 = F0/[(b)2 + (2m – k)2]1/2.

The frequency that produces the maximum amplitude is

2 = [(k/m) – (b2/2m2)].

This has the same form as the result for an LRC circuit, with

k  1/C,  m  L,  b  R.

30.	The current in an LRC circuit is

I = V0/Z,

so the maximum occurs for Z = Zmin = R:

I0 = V0/R, with 0 = 1/(LC)1/2.

To get I = !I0 , we have

Z = 2Zmin ,  or

Z2 = 4R2 = R2 + (L – 1/C)2, which gives

3R2 = (L – 1/C)2,  or  v3R = ± (L – 1/C),

which is a quadratic equation for :

± L2 – v3R — 1/C = 0,   or   L2 — v3R – 1/C = 0, with solutions ( must be positive)

 = ± v3R/2L + [(3R2/4L2) + (1/LC)]1/2.

Thus the width of the peak is

? = + – – = v3R/L.

31.	(a)	The average power dissipated is

= Irms2Z cos  = Irms2R = !I02R.

If we use I0 = V0/Z, we get

= V02R/2Z2 = V02R/2[R2 + (L – 1/C)2].

(b)	We find the frequency to make  a maximum by setting the first derivative equal to zero:

, or equivalently, ;

Because (L + 1/2C) > 0, we have

L = 1/C,  or         2 = 1/LC.

(c)	The peak for the average power, P0 = V02R/2Z02, occurs when Z0 = R.

To get   = V02R/2Z2 = !P0 = V02R/4Z02, we have

Z2 = R2 + (L – 1/C)2 = 2Z02 = 2R2,   or   R = ± (L – 1/C),

which is a quadratic equation for :

± L2 – R — 1/C = 0,   or   L2 — R – 1/C = 0, with solutions ( must be positive)

 = ± R/2L + [(R2/4L2) + (1/LC)]1/2.

Thus the width of the peak is

? = + – – = R/L.

32.	Because the loudspeaker is connected to the secondary side of the transformer, we have

ZP/ZS = (NP/NS)2;

(35103 )/(8.0 ) = (NP/NS)2, which gives NP/NS =      66.

33.	We find the equivalent resistance of the two speakers in parallel from

1/Req = (1/R1) + (1/R2) = (1/8 ) + (1/8 ), which gives Req = 4 .

Thus the speakers should be connected to the      4       terminals.

34.	Because the capacitor and resistor are in series, the impedance of the circuit is

Z = (R2 + XC2)1/2,

so the current is

I = V/Z,

and the voltage across the resistor is

VR = IR.

(a)	The reactance of the capacitor is

XC1 = 1/2pf1C = 1/2p(60 Hz)(1.210–6 F) = 2.21103  = 2.21 k.

The impedance of the circuit is

Z1 = (R2 + XC12)1/2 = [(0.800 k)2 + (2.21 k)2]1/2 = 2.35 k.

The current is

I1 = V/Z1 = (130 mV)/(2.35 k) = 55.3 A.

The voltage across the resistor is

VR1 = I1R = (55.3 A)(0.800 k) =       44 mV.

Thus the signal is reduced but not eliminated.

(b)	The reactance of the capacitor is

XC2 = 1/2pf2C = 1/2p(60,000 Hz)(1.210–6 F) = 2.21 .

The impedance of the circuit is

Z2 = (R2 + XC22)1/2 = [(800 )2 + (2.21 )2]1/2 = 800 .

The current is

I2 = V/Z2 = (130 mV)/(800 ) = 0.163 mA.

The voltage across the resistor is

VR2 = I2R = (0.163 mA)(800 ) =       130 mV.

Thus the high-frequency signal passes to the resistor.

35.	For the current and voltage to be in phase, the net reactance of the capacitor and inductor must be zero.  Thus we have

XL = XC;

2pfL = 1/2pfC,  or

f = (1/2p)(1/LC)1/2;

3360 Hz = (1/2p)[1/(0.230 H)C]1/2 , which gives C = 9.7610–9 F =      9.76 nF.

36.	Because the current lags the voltage by less than 90°, resistance must be present so this must be an

RL circuit.

We find the impedance of the circuit from

Z = Vrms/Irms = (120 V)/(5.6 A) = 21.4 .

We find the resistance from the phase:

cos  = R/Z;

cos 50° = R/(21.4 ), which gives R =        13.8 .

We find the reactance from the phase:

sin  = XL/Z;

sin 50° = XL/(21.4 ), which gives XL = 16.4 .

We find the inductance from

XL = 2pfL;

16.4  = 2p(60 Hz)L, which gives L = 4.3510–2 H =      43.5 mH.

37.	We find the reactance from

XL = Vrms/Irms = (240 V)/(22.8 A) = 10.5 .

We find the inductance from

XL = 2pfL;

10.5  = 2p(60 Hz)L, which gives L = 0.0279 H =       27.9 mH.

38.	(a)	We find the impedance from

Z = XC = 1/2pfC = 1/2p(700 Hz)(0.03810–6 F) = 5.98103  =      6.0 k.

(b)	We find the peak value of the current from

Ipeak = v2 Irms = v2(Vrms/Z) = v2(4.0 kV)/(6.0 k) =      0.95 A.

39.	At 60 Hz, the reactance of the inductor is

XL1 = 2pf1L = 2p(60 Hz)(0.620 H) = 234 .

The impedance of the circuit is

Z1 = (R2 + XL12)1/2 = [(3.5 k)2 + (0.234 k)2]1/2 = 3.51 k.

Thus the impedance at the new frequency is

Z2 = 2Z1 = 2(3.51 k) = 7.02 k.

We find the new reactance from

Z2 = (R2 + XL22)1/2;

7.02 k = [(3.5 k)2 + XL22]1/2, which gives XL2 = 6.09 k.

We find the new frequency from

XL2 = 2pf2L;

6.09 k = 2pf2(0.620 H), which gives f2 =       1.6 kHz.

40.	(a)	The reactance of the capacitor is

XC = 1/2pfC = 1/2p(60.0 Hz)(1.8010–6 F) = 1.47103  = 1.47 k.

The impedance of the circuit is

Z = (R2 + XC2)1/2 = [(8.80 k)2 + (1.47 k)2]1/2 = 8.92 k.

The rms current is

Irms = Vrms/Z = (120 V)/(8.92 k) =      13.5 mA.

(b)	We find the phase angle from

cos  = R/Z = (8.80 k)/(8.92 k) = 0.986.

In an RC circuit, the current leads the voltage, so  =      – 9.48°.

(c)	The power dissipated is

P = Irms2R = (13.510–3 A)2(8.80103 ) =        1.59 W.

(d)	The rms readings across the elements are

VR = IrmsR = (13.5 mA)(8.80 k) =      119 V;

VC = IrmsXC = (13.5 mA)(1.47 k) =      19.8 V.

Note that, because the maximum voltages occur at different times, the two readings do not add

to the applied voltage of 120 V.

41.	We find the resistance of the coil from the dc current:

R = Vdc/Idc = (36 V)/(2.5 A) =       14 .

We find the impedance from the ac current:

Z = Vrms/Irms = (120 V)/(3.8 A) = 31.6 .

We find the reactance from

Z = [R2 + XL2]1/2 ;

31.6  = [(14.4 )2 + XL2]1/2, which gives XL = 28.1 .

We find the inductance from

XL = 2pfL;

28.1  = 2p(60 Hz)L, which gives L = 0.075 H =      75 mH.

42.	(a)	At resonance we have

2pf0 = (1/LC)1/2.

The Q factor is

Q = VC/VR = IrmsXC/IrmsR = 1/2pf0CR = (LC)1/2/CR = (1/R)(L/C)1/2.

(b)	We find the inductance from the resonant frequency:

2pf0 = (1/LC)1/2;

2p(1.0106 Hz) = [1/L(0.01010–6 F)]1/2, which gives L = 2.510–6 H =      2.5 H.

We find the resistance required from

Q = (1/R)(L/C)1/2;

1000 = (1/R)[(2.510–6 H)/(0.01010–6 F)]1/2, which gives R =       1.610–2 .

(c)	For the circuit of Example 31–3, we have

Q = (1/R)(L/C)1/2 = (1/25.0 )[(30.010–3 H)/(12.010–6 F)]1/2 =      2.00.

43.	We find the impedance from the power factor:

cos  = R/Z ;

0.17 = (200 )/Z , which gives Z = 1.18103 .

We get an expression for the reactances from

Z2 = R2 + (XL – XC)2;

(1.18103 )2 = (200 )2 + (XL – XC)2, which gives XL – XC = ± 1.16103 .

When we express this in terms of the inductance and capacitance, we get

2pfL – (1/2pfC) = ± 1.16103 ;

2pf(0.020 H) – [1/2pf(5010–9 F)] = ± 1.16103 , which reduces to two quadratic equations:

0.126f 2 ± (1.16103 Hz)f – 3.183106 Hz2 = 0,

which have positive solutions of f =        2.2103 Hz, 1.1104 Hz.

44.	(a)	If we choose V = V0 sin t and assume XL > XC ,

we have the diagram shown.

(b)	We take projections on the y-axis.

From the diagram we see that

I = – I0 sin ( – t) = I0 sin (t – ),

where

cos  = VR0/V0 = R/Z;

Z2 = R2 + (XL – XC)2;  and

I0 = V0/Z.

45.	(a)	From the expression for V, we see that V0 = 0.95 V, and 2pf = 754 s–1.

For the reactances, we have

XC = 1/2pfC = 1/(754 s–1)(0.3010–6 F) = 4.42103  = 4.42 k.

XL = 2pfL = (754 s–1)(0.0220 H) = 16.6  = 0.0166 k.

The impedance of the circuit is

Z = [R2 + (XL – XC)2]1/2 = [(23.2 k)2 + (0.0166 k – 4.42 k)2]1/2 =       23.6 k.

We find the phase angle from

tan  = (XL – XC)/R = (0.0166 k – 4.42 k)/(23.2 k) = – 0.191, which gives         = – 10.8°.

(b)	The power dissipated is

P 	= Irms2R = (Vrms/Z)2R = (V0/Zv2)2R = !(V0/Z)2R

= !(0.95 V/23.6103 )2(23.2103 ) =       1.8810–5 W.

(c)	The rms current is

Irms = I0/v2 = V0/Zv2 = (0.95 V)/(23.6103 )v2 = 2.84610–5 A =       2.810–5 A.

The rms readings across the elements are

VR = IrmsR = (2.8510–5 A)(23.6103 ) =      0.66 V;

VL = IrmsXL = (2.8510–5 A)(16.6 ) =      4.710–4 V.

VC = IrmsXC = (2.8510–5 A)(4.42103 ) =      0.126 V.

46.	(a)	With XL > XC , and R = 0, the impedance of the circuit

to ac current is

Z = [(XL – XC)2]1/2 = XL – XC .

The ac current amplitude is

I0 = V20/Z = V20/(XL – XC) =       V20/(L – 1/C).

We find the phase angle from

tan   = (XL – XC)/R = (XL – XC)/0  = + 8, so         = 90°.

Thus the current is

I = I0 sin (t – 90°) = – I0 cos t.

There will be no dc current through the capacitor.

(b)	We can treat the emf as a superposition of ac and dc components.

For the dc component V1 , the voltage across the inductor is

V1L = L dI/dt = 0, so V1out = V1C = V1 , and Q1 = CV1.

At any instant the charge on the capacitor is

Q = CVout = CV1 + CV2out ,

and the current through the capacitor is

I = dQ/dt = C dV2out/dt = – I0 cos t.

When we integrate this, we get

(c)	If we ignore the sign change, the ratio of output ac voltage to input ac voltage is

V2out/V20 = 1/C(L – 1/C) = XC/(XL – XC).

This will be least when the denominator is maximal, XC « XL , and the ratio will be

V2out/V20 ˜ XC/XL « 1.

(d)	Because there is no dc current,       V1out = V1 .

47.	The impedance of the circuit to ac current is

Z = [R2 + XC2]1/2 = [R2 + (1/C)2]1/2.

The ac current amplitude is

I0 = V20/Z = V20/[R2 + (1/C)2]1/2.

We find the phase angle from

tan  = (XL – XC)/R = – XC/R.

If R » XC , then tan  ˜ 0, and  ˜ 0.

Thus the current is

I = I0 sin t.

There will be no dc current through the capacitor, and thus no appreciable dc voltage drop across the resistor, so V1out = V1C = V1 , and Q1 = CV1.

At any instant the charge on the capacitor is

Q = CVout = CV1 + CV2out ,

and the current through the capacitor is

I = dQ/dt = C dV2out/dt = I0 sin t.

When we integrate this, we get

If R » XC , then

Z = [R2 + (1/C)2]1/2 ˜ R, and the current will be

I0 = V20/Z = V20/[R2 + (1/C)2]1/2 ˜ V20/R, which will be low.

If we form the ratio of output ac voltage to input ac voltage, we get

V2out/V20 = XC/[R2 + XC2]1/2 ˜ XC/R « 1.

48.	(a)	The current in the resistor is in phase with the voltage across the resistor:

IR = V/R = (V0/R) sin t,    R = 0°.

(b)	The voltage across the inductor is

VL = V0 sin t = L dIL/dt.

When we integrate this, we get

(c)	The charge on the capacitor is

QC = CV = CV0 sin t.

Thus the current in the capacitor is

IC = dQC/dt = CV0 cos t =       CV0 sin (t + 90°) = (V0/XC) sin (t + 90°).

(d)	The total current is

I 	= IR + IL + IC = (V0/R) sin t + (V0/L) sin (t – 90°) + (CV0) sin (t + 90°)

= V0[(1/R) sin t + (C – 1/L) cos t].

We form the triangle shown with the following  values:

cos  = (1/R)/[(1/R)2 + (C – 1/L)2]1/2;

sin  = (C – 1/L)/[(1/R)2 + (C – 1/L)2]1/2;

tan  = (C – 1/L)/(1/R) = R(C – 1/L).

This lets us express the current as a combination of

trig functions:

I 	=  V0[(1/R)2 + (C – 1/L)2]1/2(cos  sin t + sin  cos t)

=       V0[(1/R)2 + (C – 1/L)2]1/2 sin (t + ), with  = tan–1 [R(C – 1/L)].

(e)	The impedance  is

Z = V0/I0 = [(1/R)2 + (C – 1/L)2]–1/2 =       R/[1 + (RC – R/L)2]1/2.

(f)	Because power is dissipated only by the resistance, the average power  is

= (I0R2/2)R = (I0R2/2)Z(power factor),

or, in terms of the voltage:

[(V0/R)2/2]R = [(V0/Z)2/2]Z(power factor),  or  V02/2R = (V02/2Z)(power factor).

Thus we see that

power factor = Z/R =       [1 + (RC – R/L)2]–1/2 = cos .

49.	We find the equivalent values for each type of element:

R = R1 + R2 ;

C = C1C2/(C1 + C2),  so  XC = (C1 + C2)/C1C2 ;

L = L1 + L2 , assuming the windings are symmetric,  so  XL = (L1 + L2).

The impedance of the circuit is

Z = [R2 + (XL  – XC)2]1/2 =       {(R1 + R2)2 + [(L1 + L2) – (C1 + C2)/C1C2]2}1/2.

50.	If there is no current in the secondary, there will be no induced emf from the mutual inductance.  We find the inductance from

Z = XL = Vrms/Irms = 2pfL;

(220 V)/(5.8 A) = 2p(60 Hz)L, which gives L =      0.10 H.

51.	We can find the resistance from the power dissipated:

P = Irms2R;

300 W = (4.0 A)2R, which gives R =       19 .

The impedance of the electromagnet is

Z = Vrms/Irms = (120 V)/(4.0 A) = 30 .

We can find the inductance from

Z = (R2 + XL2)1/2 = [R2 + (2pfL)2]1/2;

30  = {(19 )2 + [2p(60 Hz)L]2}1/2, which gives L = 6.210–2 H =       62 mH.

52.	For the inductor circuit we have

V = V0 sin t = L dI/dt + RI.

If we let I = I0 sin (t – ), this becomes

V0 sin t 	= LI0 cos (t – ) + I0R sin (t – )

= LI0(cos t cos  + sin t sin ) + I0R(sin t cos  – cos t sin )

= (I0R cos  + I0L sin ) sin t – (I0R sin  – I0L cos ) cos t.

Because this must be satisfied for any t, the equation must be satisfied separately by the sin t

and cos t terms.  Thus we get two equations from which we get I0 and :

I0R sin  – I0L cos  = 0, which gives       tan  = L/R;

V0 = I0R cos  + I0L sin  .

We form the triangle shown with the following  values:

cos  = R/(R2 + 2L2)1/2;

sin  = L/(R2 + 2L2)1/2;

tan  = L/R.

Thus I0 	= V0/(R cos  + L sin )

= V0/{R[R/(R2 + 2L2)1/2] + L[L/(R2 + 2L2)1/2]} =      V0/(R2 + 2L2)1/2.

53.	For the junction equation, we have

I = IC + IL.

For the loop equations, we have

left loop: 		V0 sin t  – IR – Q/C = 0;

outside loop:	V0 sin t  – IR – L dIL/dt = 0.

Using a trial solution, I = I0 sin (t + ), we find the

capacitor current from

Q = C[V0 sin (t)  – I0R sin (t + )];

IC 	= dQ/dt = + CV0 cos t – CI0R cos (t + )

=  + (V0/XC) cos t – (I0R/XC) cos (t + ).

We find the inductor current from

dIL/dt = (V0/L) sin t  – (I0R/L) sin (t + );

IL 	= ? (dIL/dt) dt = – (V0/L) cos t  + (I0R/L) cos (t + ) + constant

= – (V0/XL) cos t  + (I0R/XL) cos (t + ) + constant.

By using the appropriate expansion of the trigonometric functions, we have

I = I0[sin t cos  + cos t sin ],

IC =  + (V0/XC) cos t – (I0R/XC)(cos t cos  – sin t sin ),

IL =  – (V0/XL) cos t + (I0R/XL)(cos t cos  – sin t sin ) + constant.

For the junction equation to be satisfied for arbitrary t, the coefficients of sin t and cos t must satisfy the junction equation, which gives

constant = 0,

+ I0 cos  = I0R[(1/XC) – (1/XL)] sin .				(i)

+ I0 sin  = V0[(1/XC) – (1/XL)] – I0R[(1/XC) – (1/XL)] cos ,	(ii)

From (i) we get

tan  = XCXL/(XL – XC)R, which gives

Xeq = XCXL/(XL – XC).

This is the equivalent reactance of the two parallel elements.  Because the capacitor current leads the voltage and the inductor current lags the voltage, there is a p phase difference between the two, which gives the negative sign in the denominator.

The impedance of the circuit is

Z = (R2 + Xeq2)1/2 = {R2 + [XCXL/(XL – XC)]2}1/2, which can also be obtained from (ii).

When we use these results in the current equations, we obtain

I0 = V0/Z, so the three currents are

I = (V0/Z) sin (t + ),

IC = (V0/XC)[– (R/Z) cos (t + ) +  cos t],

IL = (V0/XL)[(R/Z) cos (t + ) – cos t], where

Z = {R2 + [XCXL/(XL – XC)]2}1/2, and tan  = XCXL/(XL – XC)R.

54.	We write the applied voltage as V0 sin t.

For the capacitor branch we have

V0 sin t = Q/C + I1R1, with Z12 = R12 + XC2.

Thus I1 = I10 sin (t – 1), with tan 1 = – XC/R1 .

For the inductor branch we have

V0 sin t = I2R2 + L dI2/dt, with Z22 = R22 + XL2.

Thus I2 = I20 sin (t – 2), , with tan 2 = XL/R2 .

We can find the potential difference Vab from the two paths:

Vab = – I1R1 + I2XL ;   Vab = I1XC – I2R2 .

When we eliminate I1 , we get

[1 + (R1/XC)]Vab = [XL – (R1R2/XC)]I2 .

Thus Vab = 0 if R1R2 = XCXL = L/C, independent of the frequency.