26.12:	From Ohm’s law, the voltage drop across the 6.00 Ω resistor is V = IR = (4.00 A)(6.00 Ω)=24.0 V. The voltage drop across the 8.00 Ω resistor is the same, since these two resistors are wired in parallel. The current through the 8.00 Ω resistor is then I=V/R=24.0 V/8.00 Ω=3.00 A. The current through the 25.0 Ω resistor is the sum of these two currents: 7.00 A. The voltage drop across the 25.0 Ω resistor is V = IR = (7.00 A)( 25.0 Ω) = 175 V, and total voltage drop across the top branch of the circuit is 175 + 24.0 = 199 V, which is also the voltage drop across the 20.0 Ω resistor. The current through the 20.0 Ω resistor is then I=V/R=199 V/20 Ω=9.95 A.