26.28:	Given that the full-scale deflection current is 500 μA and the coil resistance is 25.0 Ω:

a) For a 20-mA ammeter, the two resistances are in parallel:

Vc=Vs⇒IcRc=IsRs⇒500×10−6 A 25.0 Ω=20×10−3 A−500×10−6 ARs⇒Rs=0.641 Ω

b) For a 500-m voltmeter, the resistances are in series:

Vab=IRc+Rs⇒Rs⇒VabI−Rc ⇒Rs=500×10−3 V500×10−6 A−25.0 Ω=975 Ω.