26.76:	a) V=IR + IRA ⇒ R=VI− RA. The true resistance R is always less than the reading because in the circuit the ammeter’s resistance causes the current to be less then it should. Thus the smaller current requires the resistance R to be calculated larger than it should be.

b) I=VR +VRV ⇒ R=VRVIRV−V=VI − V/RV. Now the current measured is greater than that through the resistor, so R=V/IR is always greater than V/I.

c) (a): P=I2R=I2(V/I−RA)=IV−I2RA.

(b): P=V2/R=V(I − V/RV)=IV − V2/RV.