26.92:

Let current I enter at a and exit at b. At a there are three equivalent branches, so current is I/3 in each. At the next junction point there are two equivalent branches so each gets current I/6. Then at b there are three equivalent branches with current I/3 in each. The voltage drop from a to b then is V=I3 R+ I6 R + I3 R=56IR. This must be the same as V=IReq, so Req=56R.