30.69:	a) At t=0,all the current passes through the resistor R1,so the voltage vabis the total voltage of 60.0 V.

b) Point a is at a higher potential than point b. c) vcd=60.0 Vsince there is no current through R2.

d) Point cis at a higher potential than point b.

e) After a long time, the switch is opened, and the inductor initially maintains the current of iR2= εR2= 60.0 V25.0 Ω= 2.40 A. Therefore the potential between aand bis vab= −iR1= −(2.40 A) (40.0 Ω)= −96.0 V.

f) Point bis at a higher potential than point a.

g) vcd= −i(R1 + R2)= −(2.40 A) (40 Ω +25 Ω )= −156 V

h) Point dis at a higher potential than point c.