30.67:    a) Just after the switch is closed there is no current through either inductor and they act like breaks in the circuit. The current is the same through the 40.0 Ω and 15.0 Ω resistors and is equal to (25.0 V)/(40.0 Ω+15.0 Ω)=0.455 A. A1=A4=0.455 A;

A2=A3=0.

b) After a long time the currents are constant, there is no voltage across either inductor, and each inductor can be treated as a short-circuit . The circuit is equivalent to:

I=(25.0 V)/(42.73 Ω)= 0.585 A

A1 reads 0.585 A. The voltage across each parallel branch is 25.0 V−(0.585 A)(40.0 Ω)=

1.60 V. A2 reads (1.60 V)/(5.0 Ω)=0.320 A. A3 reads (1.60 V)/10.0 Ω)=0.160 A. A4reads (1.60 V)/(15.0 Ω)=0.107 A.