25.3:	a) vd=InqA=4.85 A(8.5 × 1028)(1.6 × 10−19 C)(π/4)(2.05 × 10−3 m)2)

=1.08 × 10−4 m/s

⇒ travel time =dvd =0.71 m1.08 × 10−4 m/s =6574 s =110 min

b) If the diameter is now 4.12 mm, the time can be calculated using the formula above or comparing the ratio of the areas, and yields a time of 26542 s =442 min.

c) The drift velocity depends on the diameter of the wire as an inverse square relationship.