25.75:	a) I=Σ εΣ R=12.0 V − 8.0 V10.0 Ω= 0.40 A.

b) Ptotal=I2 Rtotal=(0.40 A)2 (10 Ω) = 1.6 W.

c) Power generated in ε1, P = ε1 I = (12.0 V) (0.40 A)=4.8 W.

d) Rate of electrical energy transferred to chemical energy in ε2 P=ε2I=(8.0 V) × (0.40 A)=3.2 W.

e) Note (c)=(b) + (d), and so the rate of creation of electrical energy equals its rate of dissipation.