24.63:	a) Reducing the furthest right leg yields C=16.9 μF+ 16.9 μF+ 16.9 μF−1=  2.3 μF= C13. It combines in parallel with aC2⇒C=4.6 μF + 2.3 μF=6.9 μF=C1. So the next reduction is the same as the first:C=2.3 μF=C13.And the next is the same as the second, leaving 3C1’s in series soCeq=2.3 μF=C13.

b) For the three capacitors nearest points a and b:

QC1=CeqV=(2.3 × 10−6 F)(420 V)=9.7 × 10−4 C

and QC2=C2V2=(4.6 ×10−6 F)(420 V)/3=6.44 × 10−4 C.

c) Vcd=134203 V=46.7 V,since by symmetry the total voltage drop over the equivalent capacitance of the part of the circuit from the junctions between a, c  and d, b is 4203 V, and the equivalent capacitance is that of three equal capacitors C1in series. Vcd is the voltage over just one of those capacitors, i.e., 1/3of 4203 V.