24.60:	a) With the switch open: Ceq=13 μF + 16 μF−1 + 13 μF + 16 μF−1=4.00 μF

⇒ Qtotal=CeqV=(4.00 μF) (210 V) =8.4 × 10−4 C. By symmetry, each

capacitor carries 4.20 10−4 C.  The voltages are then just calculated via V=Q/C.

So: Vad=QC3=140 V, and Vac=QC6=70 V⇒Vcd=Vad − Vac =70 V.

b) When the switch is closed, the points c and d must be at the same potential, so the equivalent capacitance is:

Ceq=1(3 + 6) μF + 1(3 + 6) μF −1=4.5 μF.

⇒Qtotal=CeqV=(4.50 μF) (210 V)=9.5 × 10−4 C,and each capacitor has the same potential difference of 105 V (again, by symmetry)

c) The only way for the sum of the positive charge on one plate of C2and the negative charge on one plate of C1to change is for charge to flow through the switch. That is, the quantity of charge that flows through the switch is equal to the charge in Q2 − Q1=0. With the switch open, Q1= Q2 and Q2 − Q1=0. After the switch is closed, Q2 − Q1=315 μC; 315 μC of charge flowed through the switch.