30.66:	(a) Initially the capacitor behaves like a short circuit and the inductor like an open circuit. The simplified circuit becomes

i=εR=75 V150 Ω=0.500 A

V1=Ri=(50 Ω)(0.50 A)=25.0 VV3=0, V4=(100 Ω)(0.50 A)=50.0 VV2=V4 (in parallel)=50.0 VA1=A3=0.500 A, A2=0

(b) Long after S is closed, capacitor stops all current. Circuit becomes

V3=75.0 V and all other meters read zero.

(c) q=CV=(75 nF)(75 V)=5630 nC, long after S is closed.