20.51:	a) The cylinder described contains a mass of air m=ρ(πd2/4)L, and so the total kinetic energy is K=ρ(π/8)d2Lv2. This mass of air will pass by the turbine in a time t=L/v, and so the maximum power is

P=Kt=ρ(π/8)d2v3.

Numerically, the product ρair(π/8)≈0.5kg/m3=0.5 W⋅s4/m5.

b) v=P/ekd213=(3.2×106 W)/(0.25)(0.5 W⋅s4/m5)(97 m)213=14m/s=50 km/h.

c) Wind speeds tend to be higher in mountain passes.