26.64:	First recognize that if the 40 Ω resistor is safe, all the other resistors are also safe.

I2R=P→I2(40 Ω)=1 WI=0.158 A

Now use series / parallel reduction to simplify the circuit. The upper parallel branch is 6.38 Ω and the lower one is 25 Ω. The series sum is now 126 Ω. Ohm’s law gives

ε=(126 Ω)(0.158 A)=19.9 V