30.65:    a) Just after the switch is closed the voltage V5 across the capacitor is zero and there is also no current through the inductor, so V3=0. V2 + V3=V4=V5, and since V5=0 and V3=0, V4 and V2 are also zero. V4=0 means V3 reads zero.

V1 then must equal 40.0 V, and this means the current read by A1 is (40.0 V)(50.0 Ω)=0.800 A.

A2+A3+A4=A1, but A2=A3=0  so  A4=A1=0.800 A.

A1=A4=0.800 A; all other ammeters read zero.

V1=40.0 V and all other voltmeters read zero.

b) After a long time the capacitor is fully charged so A4=0. The current through the inductor isn’t changing, so V2=0. The currents can be calculated from the equivalent circuit that replaces the inductor by a short-circuit.:

I=(40.0 V)(83.33 Ω)=0.480 A; A1 reads 0.480 AV1=I(50.0 Ω)=24.0 V

The voltage across each parallel branch is 40.0 V−24.0 V=16.0 V

V2=0, V3=V4=V5=16.0 V

V3=16.0 V means A2 reads 0.160 A. V4=16.0 V means A3 reads 0.320 A. A4 reads zero. Note that A2+A3=A1.c) V5=16.0 V so Q=CV=(12.0 μF)(16.0 V)=192 μC

d) At t = 0 and t → ∞, V2=0. As the current in this branch increases from zero to 0.160 A the voltage V2 reflects the rate of change of current.