26.89:	a) Using Kirchhoff’s Rules on the circuit we find:

Left loop:	92 −140I1 − 210I2 + 55 =0⇒ 147 − 140I1 − 210I2=0.

Right loop:	   57 −35I3 − 210I2 + 55 =0 ⇒ 112− 210I2 − 35I3=0.

Currents:	  					⇒ I1 − I2 + I3=0.

Solving for the three currents we have:

I1=0.300 A, 	I2=0.500 A,	I3=0.200 A.

b) Leaving only the 92-V battery in the circuit:

Left loop:	92 −140I1 − 210I2=0.

Right loop:	       −35I3− 210I2 =0.

Currents:	            I1− I2 + I3=0.

Solving for the three currents:

I1=0.541 A,	I2=0.077 A,	I3=−0.464 A.

c) Leaving only the 57-V battery in the circuit:

Left loop:	       140I1+210I2=0.

Right loop:	57 − 35I3 − 210I2=0.

Currents:	           I1− I2 + I3=0.

Solving for the three currents:

I1=−0.287 A,	I2=0.192 A,	I3=0.480 A.

d) Leaving only the 55-V battery in the circuit:

Left loop:	55 −140I1 − 210I2=0.

Right loop:	 55−35I3−210I2=0.

Currents:	           I1−I2 + I3=0.

Solving for the three currents:

I1=0.046 A,	I2=0.231 A,	I3=0.185 A.

e) If we sum the currents from the previous three parts we find:

I1=0.300 A,	I2=0.500 A,	I3=0.200 A, just as in part (a).

f) Changing the 57-V battery for an 80-V battery just affects the calculation in part (c). It changes to:

Left loop:	      140I1 + 210I2=0.

Right loop:	80 −35I3 − 210I2=0.

Currents:	          I1− I2 + I3=0.

Solving for the three currents:

I1=−0.403 A,	I2=0.269 A,	I3=0.672 A.

So the total current for the full circuit is the sum of (b), (d) and (f) above:

I1=0.184 A,	I2=0.576 A,	I3=0.392 A.