26.65:  The 20.0-Ω and 30.0-Ω resistors are in parallel and have equivalent resistance 12.0 Ω. The two resistors R are in parallel and have equivalent resistance R/2. The circuit is equivalent to

a) 	Va − (5.00 A)(12.0 Ω) − (5.00 A)(18.0 Ω) − 20.0 V=Vc

Vc − Va =20.0 V + 90.0 V + 60.0 V=170.0 VVb− Va=Vba=16.0 VX − Vba=170.0 V so X=186.0 V,withtheupperterminal +

b) I1=(16.0 V)/(8.00 Ω) =2.00 A

The junction rule applied to point a gives I2+I1 =5.00 A, so I2 =3.00 A. The

current through the 200.0 V battery is in the direction from the − to the + terminal, as shown in the diagram.

c) 200.0 V− I2(R/2) =170.0 V

(3.00 A)(R/2) =30.0 V so R =20.0 Ω