7.57:    The two design conditions are expressed algebraically as ky=f+mg=3.66×104 N (the condition that the elevator remains at rest when the spring is compressed a distance y; y will be taken as positive) and 12mv2+mgy−fy=12kx2 (the condition that the change in energy is the work Wother=−fy). Eliminating y in favor of k by y=3.66×104 Nk leads to

12(3.66×104 N)2k+(1.70×104 N)(3.66×104 N)k

=62.5×104 J+(1.96×104 N)(3.66×104 N)k.

This is actually not hard to solve for k=919 N/m, and the corresponding x is 39.8 m. This is a very weak spring constant, and would require a space below the operating range of the elevator about four floors deep, which is not reasonable.  b) At the lowest point, the spring exerts an upward force of magnitude f+mg. Just before the elevator stops, however, the friction force is also directed upward, so the net force is (f+mg)+f−mg=2f, and the upward acceleration is 2fm=17.0 m/s2.