27.41:	a) The force on the straight section along the –x-axis is zero.

For the half of the semicircle at negative x the force is out of the page. For the half of the semicircle at positive x the force is into the page. The net force on the semicircular section is zero.

The force on the straight section that is perpendicular to the plane of the figure is in the –y-direction and has magnitude F=ILB.

The total magnetic force on the conductor is ILB,in the − y-direction.

b) If the semicircular section is replaced by a straight section along the x-axis, then the magnetic force on that straight section would be zero, the same as it is for the semicircle.