25.34:	a) A voltmeter placed over the battery terminals reads the emf: ε=24.0 V.

b) There is no current flowing, so Vr=0.

c) The voltage reading over the switch is that over the battery: Vs=24.0 V.

d) Having closed the switch:

I=24.0 V/5.88 Ω=4.08 A ⇒ Vab=24.0 V − (4.08 A)(0.28 Ω)=22.9 V.

Vr=IR=(4.08 A)(5.60 Ω)=22.9 V.

Vs=0, since all the voltage has been “used up” in the circuit. The resistance of the switch is zero so Vs=IR=0.