12.63:     One can solve this problem using energy conservation, units of J/kg for energy, and basic concepts of orbits.E=K+U, or −GM2a=12v2−GMr,where E,K and U are the energies per unit mass,v is the circular orbital velocity of 1655 m/s at the lunicentric distance of 1.79×106 m. The total energy at this distance is−1.37×106J/Kg. When the velocity of the spacecraft is reduced by 20 m/s, the total energy becomes

E=12(1655 m  s− 20 m  s)2−(6.673×10−11 N⋅m2kg2) (7.35×1022 kg)(1.79×106 m),

or E=−1.40×106J/kg. Since E=−GM2a, we can solve for a,a=1.748×106 m,the semi –major axis of the new elliptical orbit. The old distance of 1.79×106 m is now the apolune distance, and the perilune can be found from a=ra+rp2,rp=1.706×106 m.Obviously this is less than the radius of the moon, so the spacecraft crashes! At the surface, U=−GMRm or, U=−2.818×106J/kg.

Since the total energy at the surface is −1.40×106J/kg,the kinetic energy at the surface is 1.415×106J/kg. So,12v2=1.415×106J/kg, or v=1.682×103m/s =6057 km/h.