24.28:	a) Q=CV0.

b) They must have equal potential difference, and their combined charge must add up to the original charge. Therefore:

V=Q1C1=Q2C2and also Q1 + Q2=Q=CV0

C1=C and C2=C2 so Q1C=Q2(C/2) ⇒ Q2=Q12⇒Q=32Q1 ⇒ Q1=23Q so V=Q1C=23QC=23V0

c) U=12Q12C1 + Q22C2=12(23Q)2C +2(13Q)2C=13Q2C=13CV02

d) The original U was U=12CV02 ⇒ ΔU=−16CV02.

e) Thermal energy of capacitor, wires, etc., and electromagnetic radiation.