25.55:	With the voltmeter connected across the terminals of the battery there is no current through the battery and the voltmeter reading is the battery emf; ε=12.6 V.

With a wire of resistance Rconnected to the battery current I flows and ε − Ir −IR =0

Call the resistance of the 20.0-m piece R1; then the resistance of the 40.0-m piece is R2=2R1.

ε − I1r−I1R1=0;	12.6 V−(7.00 A)r−(7.00 A)R1=0

ε − I2r − I2(2R1)=0;	12.6 V − (4.20 A)r − (4.20 A)(2R1)=0

Solving these two equations in two unknowns gives R1=1.20Ω. This is the resistance of 20.0 m, so the resistance of one meter is [1.20Ω(20.0m)] (1.00m)=0.060Ω