17.112:    Denote the quantites for copper, brass and steel by 1, 2 and 3, respectively, and denote the temperature at the junction by T0.

a)H1=H2+H3, and using Eq.17.21 and dividing by the common area,

k1L1100°C−T0=k2L2T0+k3L3T0.

Solving for T0 gives

T0=k1/L1k1/L1+k2/L2+k3/L3100°C.

Substitution of numerical values gives T0=78.4°C.

b) Using H=kALΔT for each rod, with ΔT1=21.6C°,  ΔT2=ΔT3=78.4°C gives H1=12.8 W, H2=9.50 W and H3=3.30 W . If higher precision is kept, H1 is seen to be the sum of H2 and H3.