11.71:     a) Consider the forces on the roof

V and H  are the vertical and horizontal forces each wall exerts on the roof.

w=20,000N is the total weight of the roof.2V=w  so V=w/2

Apply Στ=0 to  one half of the  roof, with the axis along the line where the two halves join. Let each half have length L.        (w/2)(L/2)(cos 35.0°)+HLsin35.0°−VLcos35.0°=0        L divides out, and use V=w/2        Hsin35.0°=14wcos35.0°        H=w4tan35.0°=7140 N

By Newton’s 3rd law, the roof exerts a horizontal, outward force on the wall. For torque about an axis at the lower end of the wall, at the ground, this force has a larger moment arm and hence larger torque the taller the walls.

b)

Consider the torques

on one of the walls.

H is the horizontal force exerted by the roof, as considered in part (a). B is the horizontal force exerted by the buttress. Now the angle is 40°,so H=w4tan40°=5959 N

τ=0, axis at the groundH(40 m)−B(30 m)=0and B =7900 N.