17.67:     (Although it may be easier for some  to solve for the heat flow per unit area, part (b), first  the method presented here follows the order in the text.)    a) See Example 17.13; as in that  example, the area may be divided out, and solving for temperature T at the boundary,

T=kfoam/LfoamTin+kwood/LwoodToutNkfoam/Lfoam+kwood/Lwood

0.010 W/m⋅K/2.2 cm19.0° C+0.080 W/m⋅K/3.0 cm−10.0° C0.010 W/m⋅K/2.2 cm+0.080 W/m⋅K/3.0 cm

=−5.8°C.

Note that the conversion of the thickness to meters was not necessary.  b) Keeping extra figures for the result of part, (a), and using that result in the temperature difference across either the wood or the foam gives

HfoamA=HwoodA=0.010 W/m⋅K19.0° C−−5.767° C2.2×10−2m

=0.080 W/m⋅K−5.767°C−−10.0°C3.0×10−2m

=11 W/m2.