31.36:	a) Ztweeter=R2+(1/ωC)2

b) Zwoofer=R2+ωL2

c) If Ztweeter=Zwoofer, then the current splits evenly through each branch.

d) At the crossover point, where currents are equal:

R2+1/ωC2=R2+ωL2⇒ω=1LC.