30.73:	a) Just after the switch is closed there is no current in the inductors. There is no current in the resistors so there is no voltage drop across either resistor. A reads zero and V reads 20.0 V.

b) After a long time the currents are no longer changing, there is no voltage across the inductors, and the inductors can be replaced by short-circuits. The circuit becomes equivalent to

Ia= (20.0 V)/(75.0 Ω)=0.267 A

The voltage between points aand bis zero, so the voltmeter reads zero.

c) Use the results of problem 30.49 to combine the inductor network into its equivalent:

R=75.0 Ω is the equivalent resistance.

Eq.(30.14)says i= (ε/R)(1 − e−t/τ),with τ = LR= (10.8 mH)/(75.0 Ω)= 0.144 msε=20.0 V, R= 75.0 Ω, t= 0.115 ms, so i=0.147 AVR=iR=(0.147 A)(75.0 Ω)= 11.0 V20.0 V −VR− VL=0 so VL=20.0 V − VR= 9.0 V