10.33:     a) τ=Pω=(150× 103 W)/(400 rev/min)π30rad/srev/min=358 N⋅m.

b) If the tension in the rope is F,F=w  and so w=τR=1.79×103  N.

c) Assuming ideal efficiency, the rate at which the weight gains potential energy is the power output of the motor, or wv=P, so v=P/w=83.8 m/s.Equivalently, v=ωR.