10.54:    a) The moment of inertia is not given, so the angular acceleration must be found from kinematics;

α=2θt2=2srt2=25.00 m0.30 m2.00 s2=8.33  rads2.

b) αt=8.33 rad/s22.00 s=16.67 rad/s.

c) The work done by the rope on the flywheel will be the final kinetic energy;          K=W=Fs=40.0 N5.0 m=200 J.

d)			I=2Kω2=2200 J16.67 rad/s2=1.44 kg⋅m2.