26.85:	a) We will say that a capacitor is discharged if its charge is less than that of one electron, The time this takes is then given by:

q=Q0e−t/RC⇒t=RC ln (Q0/e)⇒t=(6.7 × 105 Ω)(9.2 × 10−7 F) ln (7.0 × 10−6 C/1.6 × 10−19 C)=19.36 s,

or 31.4 time constants.

b) As shown in (a), t=τ ln (Q0/q), and so the number of time constants required to discharge the capacitor is independent of R and C, and depends only on the initial charge.